MacStereology

from

Ranfurly MicroSystems

Version 2.8

A Package for the Apple Macintosh to Quantify and Analyse Images,

to Acquire Data using a Digitising Tablet and

to Display and Measure 3-D Reconstructions from Serial Sections

© Copyright V. A. Moss, 1993, Ranfurly MicroSystems. 
20 St Omer Rd, Cowley, Oxford OX4 3HB, England. Tel (44) 01865 777486.(Version 2 – 6th edition)

email : cmpowell@rmplc.co.uk

Contents

Section 1 – Getting started	4

1.1	What does MacStereology do? 	4

1.2	File Icons 	4

1.3	Hardware 	4

1.3.1	Adjusting memory size

1.3.2	Video frame input

1.3.3	Serial-line digitising tablet

1.3.4	ADB digitising tablet

1.3.5	Hard copy

1.4	General operation of MacStereology	6

Section 2 – Test data files	8

2.1	3-D test files	8

2.2	Analysing binary images	10

2.3	Analysing a sequence of Pict files.	12

2.3.1	Automatic analysis of a sequence of Pict files.

2.3.2	Analysis to show partially merging surfaces.

2.3.3	Creating Pict from a data file

2.3.4	Other options for Reading and Analysing images

2.4	Making a Movie	15

2.4.1	Muybridge photos

2.4.2	Animated 3-D reconstructions

2.4.3	Animation to show stages of reconstruction

Section 3 – Testing the digitising tablet	17

3.1	Setting up the tablet	17

3.2	Using the tablet	17

3.3	Using a microscope with a digitising tablet	17

3.4	Using the data	18

Section 4 – Using MacStereology	19

4.1	Data Collection using a digitising tablet	19

4.1.1	Using the cursor buttons

4.1.2	Drawing an object

4.2	Point mode	20

4.3	Identifiers for objects and frames	21

4.4	Correcting errors	22

4.5	Input from Binary Pict files	22

4.6	Measurement and Stereology	23

Section 5 – 3-D Reconstruction	24

5.1	Formatting data	24

5.2	Positioning the sections	24

5.2.1	Input from Binary Pict files

5.2.2	Using a digitising tablet

5.3	Identifier numbers	25

5.4	Subsequent sections	25

5.5	Displaying a 3-D Reconstruction	25

5.6	Rotating and Tilting a reconstruction	26

5.7	Linking objects	27

5.8	Printing 3-D reconstructions on a LaserWriter and saving Postscript files	28

Section 6 – Reference section	30

6.1	Apple menu – 	30

About Ster

Ster help

6.2	File menu	31

New	Page setup

Open	Print

Close	I-O config

Save	Mac config

Save as	File options

Save template	Defaults

Text file	Window/Full screen

Pict file	Quit

6.3	Image menu	35

Copy image/Copy selection	Read options

Paste image	Analyse

Clear image	Analyse options

Save image/Save selection	Point mode

Read image	Calibrate

Read binary	Movie

6.4	Tablet menu	38

Step size/Point diameter	Show X-Y

Tablet scale	IDs in order

Tablet wait	Point mode

Input Reset	Beep points

Offset

6.5	Options menu	40

Object ID	Auto clear

Frame ID	Fill objects

Magnification	Pen size

Thickness	Label objects

End frame	Draw frame

Delete object	Redraw

Delete frame	Measure

Clear screen	Measure options

6.6	Units menu	42

6.7	Data menu	43

Data format	Start movie/Add to movie

Edit data	Delete image

Precision	Run movie

List data	Erase movie

Volumes	Sequence

Font	Oscillate

Font size	Fade

Colour table

6.8	3-D menu	46

Wireframe	Linkage

Layers	Align

Surface	Clip

Viewpoint	Output

Lighting	Goodies

Position	Reset 3D

Select	Show page

Order	Black B-G/White B-G

Section 7 – Technical details	53

7.1	Items which appear in several places	53

7.2	Data structure	54

7.3	Default settings	54

7.4	MacStereology defaults	55

7.5	History		55

7.6	Glossary	56

.c.Section 1 – Getting started

This manual is divided into sections, and some information is repeated to reduce the need to jump around the manual.  It is designed for the new user who has some experience of Macintosh applications and image analysis.  If you are familiar with Macintosh applications go to Section 2 and try the test files.  Sections 4 and 5 are for when you start to digitise your own data and Section 6 describes in detail what each menu item does.  Section 7 can be ignored by all but the keenest users.

.c.1.1	What does MacStereology do?

MacStereology is a package designed to collect and manipulate data from binary images (Pict) or input by hand from a digitising tablet.  The area of a profile of a cell or a peak in a chart recording or the perimeter of a line can be measured.  Alternatively, the co-ordinates of describing profiles on serial sections can be saved in order to use the data to make 3-D reconstructions that can be viewed from any angle with the parts shown in various plotting modes to see internal detail.  MacStereology can display stereo pairs, animate sequences of reconstructions and do translucent reconstructions.  Volumes and surface areas can be calculated.

MacStereology is not a full image processing package as it is complemented by many other software packages, including the ‘Image’ free package (available from on line via alw.nih.edu).

.c.1.2	File Icons

MacStereology uses several icons to identify the application and its resulting files.  All of the icons contain the reconstruction of a sliced up cube:–

|  | MacStereology application. Double click this icon to start. |
| --- | --- |

|  | Data file containing co-ordinates suitable for 3-D reconstruction. Double click to open file or stationary pad.. |
| --- | --- |

|  | Data file containing summary of data only suitable for stereology §. Double click to open file or stationary pad. |
| --- | --- |

|  | Text file containing values from summary, co-ordinates or PostScript graphics data *. |
| --- | --- |

|  | Pict file containing either vectors (c.f. MacDraw) or pixels (c.f. MacPaint) ‡. |
| --- | --- |

|  | A sequence of pictures saved as a Pics file, suitable for some application that show animation. |
| --- | --- |

|  | Data pad or template containing details for data collection, display, ID names and colours. When opened a copy is taken and the pad remains unchanged. |
| --- | --- |

§	Summary (Ster) files cannot be used for 3-D reconstructions.

*	Text files are only for export to other applications and cannot be re-entered into MacStereology.

‡	Pict files are mainly for export to other applications, although MacStereology can read them (using the system palette rather than their own palette)

With System 7 a file can be turned into Stationary pad, in which case it shows a modified ican.

.c.1.3	Hardware

MacStereology is designed to run on any Apple Macintosh from a 1 Mbyte Macintosh Plus or any of the Macintosh II series using operating System 6 or 7.  However, since most Macintoshes now have at least 2 Mbyte of memory, several options have been expanded to make use of this.  The default memory size is 1.5 Mbytes, but this may usefully be expanded for larger data files, complex surface presentation, animation or a Macintosh with a large screen.  For full colour display an 8-bit colour video card (256 colours) is preferable, but quite acceptable colour can be obtained with a 4-bit display (16 colours) for simple graphics or grey tone displays.  It can be used with 32-bit colour, but this may be slower than 8-bit colour, without improving the graphics.  The minimum requirement is 1 Mbytes memory, but this may greatly slow down the application and it may be inadequate for routines such as surface reconstruction.

.c.1.3.1	Adjusting memory size

The default application size can be seen from the Get Info dialogue (File menu from desk top) and you can change the Memory size.  MacStereology has been tested with several large screens although you will probably need to increase the application size to accommodate the extra memory required for screen refresh.  The application size is only used with System 7 or MultiFinder; with Finder an application uses all the free memory of your Macintosh.

MacStereology 1.5 Mbyte includes 300k for the screen refresh of the normal Macintosh II colour monitor (640 x 512 pixels 8-bits per pixel).  Calculate your screen size = width x height x (number of bits/pixel) ÷ 8.  The Macintosh Plus screen is 512 x 342 x 1 ÷ 8 = 21,888 bytes (i.e. about 22 kbyte) which means you could reduce the application size by almost 300k if you are limited for memory.  MacStereology can be run without this buffer, but it would then have to recalculate the display after showing a dialogue.

I suggest the following minimum memory allocations for  various monitors:–

1000 kbyte  Mac Plus, Classic, etc.	(i.e using B&W only, NOT colour or grey tones)

1500 kbyte  12 &13 inch colour/grey tones	(8-bit, 256 colours/grey tones)

1500 kbyte  A4				(4-bit, 16 grey tones)

2000 kbyte  16 colour			(8-bit, 256 colours/grey tones)

2500 kbyte  13 inch 24-bit colour		(millions of colours)

2500 kbyte  A3 colour/grey tones		(8-bit, 256 colours/grey tones)

5500 kbyte  A3 colour			(24-bit, millions of colours)

If you have plenty of memory available an extra 500 kbyte will ease any memory problems.  This would reduce the need to swap bits of program out of memory and hence slightly would speed the program.

.c.1.3.2	Video input

MacStereology can process binary images as Pict files.  These files can be captured from a TV input through a frame grabber on a Macintosh II, or a flat-bed scanner, such as the Apple Scanner, which can save grey-tone or binary files.  Files should be saved at the screen display resolution (normally 75 lines/inch)  A typical frame grabber is the Data Translations DT2255 that grabs a 640 x 480 pixel (US) or 768 x 512 pixels (PAL) image.  Simple image grabbing software comes with these cards, but there are many image processing packages available.  Such image processing software can be used to segment a grey tone or colour image into a binary image suitable for analysis by MacStereology.

.c.1.3.3	Serial-line digitising tablet (Plugs into printer or modem ports)

If using an older Macintosh (e.g. Macintosh Plus), that does not have the ADB (Apple Data Bus) type keyboard and mouse, it is essential to use a serial-line digitising tablet.  MacStereology is designed for use with MacTablet, which is a Summagraphics MM 1201 (12x12 inch) tablet.  Other digitising tablets with a serial output could be used, although the input routine would probably need to be modified (an alternative digitising tablet protocol is available on request).

A four-button cursor is preferable, but not essential.  There can be problems connecting this tablet with the round 8-pin serial socket of the Macintosh (a 10 kΩ leakage resistor on the TX+ and RX+ helps).  MacTablet comes with software to enable the tablet to be used as an alternative to the mouse.  Unfortunately this only works with a very old operating system and was never very satisfactory.  However, MacStereology does not use this software as the tablet is configured to work quite separately from the mouse.

.c.1.3.4	ADB digitising tablet (Plugs into keyboard or mouse sockets)

MacStereology should configure itself to any ADB digitising tablet, provided it uses the standard Macintosh protocol released in 1990 (for older ADB tablets consult supplier for update).  These tablets are designed to be used as an alternative to the mouse, but can also be reconfigured as a separate digitising tablet (probably via the Control Panel in the Apple  menu).  The tablet uses the same ports as the keyboard and mouse of a Macintosh II or SE, but cannot be used with a Macintosh Plus.  It reads the tablet size and resolution then scales data accordingly.  If it fails to get the appropriate co-ordinates from the system it will use scale factors in the MacStereology resources.

.i.Follow the instructions by installing the INIT in the system folder and ensure the digitising tablet plugged in before starting up.  If input from an ADB tablet is selected by MacStereology, it disconnects the tablet from the mouse but returns it, on quitting, to be an alternative to the mouse.  Whilst using the tablet it will switch back to a mouse if its cursor moves into the menu bar.  Note that some tablets still work even if its software not installed, but it cannot then be used independently by MacStereology.

The Kurta IS/ADB tablets or GTCO Macintizer are typical tablets of this type.  However if MacStereology fails to find a tablet using the standard ADB protocol it will try to configure itself for the Summagraphics Bit Pad Plus digitising tablets§.

MacStereology offers you the choice of either the serial line or ADB tablet of digitising tablets, and you can even plug them both in at the same time!  A serial-line tablet is set to ‘switched stream’ mode so that it only transmits data when a button on the cursor or pen is pressed.  Data is buffered and processed during any free time.  An ADB tablet transmits continuously and is being sampled at about 50 co-ordinates per second and the co-ordinates are processed as they are collected.  With a Macintosh II this is no problem, but a Macintosh SE may be a bit slow, particularly if Show X-Y is selected.  The separate power supply of serial-line tablet is an advantage if you are worried about overloading the power supplies of your Macintosh.

.c.1.3.5	Hard copy

Hard copy can be printed using any printer (either directly connected or via AppleTalk, and probably using a print spooler).  Fine detail output to a PostScript printer uses half tones (c.f. a newspaper photograph).  For better quality print a large copy then photo reduce the print to half size.  Colour PostScript output can be selected.  There are now a range of devices for making colour prints or 35 mm slides.

Graphics can also be plotted using a digital plotter (HP 7475A or equivalent).  Other digital plotters could be configured with minor program changes.  Since the Macintosh has only two serial ports, it may be necessary to juggle the configuration if using the serial-line tablet, printer and plotter together.

For pictures of 3-D reconstructions I normally photograph the monitor with a 35 mm SLR camera on a tripod using a long focal length lens 4 feet from the monitor (to avoid the curvature of the screen) and a 1/2 second exposure (to avoid frame scan lines).  Photograph two views at 5° angle .i.rotation; difference to produce a stereo pair.

.c.1.4	General operation of MacStereology

MacStereology ;follows the normal Macintosh rules.  Listed here are some general points on operating the program.  Check your “Introduction to the Macintosh” for more details of using the mouse, disks and organising files, etc.

1.	To start the program use the mouse to double click the MacStereology icon or double click a data file which was previously generated by MacStereology.  The data knows which application it should use and will carry out the same sequence of actions as if you had started MacStereology then opened the data file.  Only one data file can be open at a time.

2.	All the data, and most other things you specified whilst using MacStereology, are saved in the data file.  Any new data or changes to an existing data file can be Saved, although you can Don’t Save changes.  Remember:– if at the start of data collection you do not choose to save the data to disk, there is the possibility of losing your data if there is a power failure.

3.	Select options in the menus using the mouse.  Some of the commonly used options have a keyboard equivalent that you can see in the menu as a Command (⌘) character followed by a letter.  This provides a short cut for selecting options – just type the letter.  (Since there is no general text input into MacStereology you do not need to press the command key ⌘ and the letter).

Pressing the Command (⌘) key also modifies some actions, such as dragging the position of graphics rather than selecting an area of the screen or changing the plotting mode for all identifiers in the Select dialogue.  Within dialogues pressing the Command (⌘) key and clicking will centre the display on that point.  Pressing the Option and Shift keys and clicking will shrink or enlarge all displays by a factor of 2 (position only approximate for 3-D reconstructions).

The ‘+’, ‘–’, ‘<‘ and ‘>‘ keys are used for moving through the display of frames in an existing data file (Section 2.3.1), but are used to change ID whilst collecting data (Section 4.3).

The arrow keys (←↑↓→) are used to move binary images, with the Shift, Option, Command and Control keys modifying the size of the step (Section 4.5).

4.	Items in menu which are not valid are shown dimmed and cannot be selected, e.g. Close when no file is open.

5.	As in any Macintosh application, there are several routes to doing any particular task. If you select an option, it will, if necessary, call other options (e.g. selecting Print will automatically call Page setup if that has not been already specified).

6.	When an item in a menu is followed by the diacritical (…) it indicates that a dialogue window will be displayed which will offer you several options:–

a.	Click the controls, enter text or numbers then select the appropriate exit button (normally OK or Cancel).  If there is no exit button (e.g. Help), just click the mouse anywhere or press any key.  During Printing, pressing Command-period (⌘ .) keys will abort the printing.

b.	The default exit option is highlighted (usually OK, but sometimes Cancel if your action is unexpected or may have serious consequences).  Pressing the Return or Enter key is the same as clicking the default button.

c.	There is always a Cancel button if you change your mind – either you do not want that option or you want to undo any changes you have made in that dialogue.  If you just want to see the values of some parameters, display the appropriate dialogue, e.g. Edit data, then exit with the Cancel button (using OK here is safe, but it may later force the data to be replotted or reSaved to disk).  The Escape key also acts as Cancel.

d.	Text or numbers that can be changed are shown inside a box.  The first item of editable text in a dialogue box is highlighted.  A new value for this one can be typed directly (making the old highlighted value disappear).  To change the others click or double click the text, then type the new value.  Using the tab key will enable you to jump around all the items of editable text.

The size of the box gives an idea how long the text can be (although you can often safely enter longer text).  All numerical values are checked and redisplayed appropriately.  Values may be entered as a decimal (12.34) or integer (12) value (the thousand divider in numbers is ignored).  [N.B. Punctuation corresponds to the nationality of the operating system so British or US 12,345.6 becomes 12.345,6 with the Dutch operating system].  Illegal characters in text are ignored.  Values can be entered including powers of 10 (e.g. 1.23e7) and values will be displayed in this format if the value is too large or too small.  The number of significant figures of the display can be varied.

In some dialogues the values are used immediately (e.g. an ID number of an object is converted to its name), but most are not interpreted until you click another item or press tab or return.  The values you specify usually remain until you change them; an exception is 3-D output to a printer will default back to the Macintosh screen.  For convenience, some of the items that are used by several operations (e.g. Magnification) appear in several dialogues and it does not matter which one is changed as they all refer to the same variable (see Section 7.1).

e.	When a value is highlighted and it has  and  boxes, you can click on the arrows to increase or decrease the value.  Holding down the mouse button will scroll rapidly through the values until it reaches the limit.  You can also use the ‘+’ or ‘–’ keys to scroll through the values (‘=’ key can be used instead of ‘+’).  The speed of scrolling is determined from your key delay and repeat rate that you chose in Keyboard setup (via Control Panel in  menu).

f.	Inactive options in a dialogue will be dimmed.  If they are invalid they cannot be edited, but where appropriate clicking will make active that option.

g.	Dialogues can be moved by dragging the title bar.  Dialogues showing images will expand if you have a larger screen.

7.	The cursor reflects what actions can be done.  The normal arrow cursor is used to select menu items or draw a range.  The watch cursor indicates the program is busy, the hand cursor () that the graphics can be dragged (may need to press ⌘ key), the tablet cursor () whilst entering data from a tablet, the 3-D cursor () whilst doing 3-D reconstructions and there are several other cursors that are explained later.

8.	The MacStereology program is not particularly large, but it is not all resident in memory all the time.  This means that both the program and the system should be in a disk drive all the time.  By giving the program a larger allocation of memory less switching will occur so the program will run faster.

For a Macintosh with only 2 floppy drives, I suggest keeping MacStereology on the system disk (configure a minimum system and remove all spare fonts, except Geneva 9, 10 and 12 pt) and using the second drive for the data disk.  Never use more discs than you have disk drives or you will be spending much of your time swapping discs!  The data collected can be displayed on any other Macintosh.

9.	Ensure the operating system is appropriately configured for your Macintosh (System 6 or 7).  The program knows about the screen sizes and can use colours, but ignores the floating point chip since calculation is faster in fixed point arithmetic.  Remember to make a back-up copy of both the program and your data discs.  During data collection the full data file is automatically saved after every frame, so if there is a crash you should not lose too much data.

10. Future versions of the program will be compatible with data collected on any older version.

The demo version may be freely copied and circulated.  The demo version is the full program with the exception o0f the routines to create new data files.  It is time limited to prevent old versions remaining in circulation, but updates are available.  Contact by email cmpowell@rmplc.co.uk .  A demo version of MacStereology, and this manual is available from NIH Image by FTP from zippy.nimh.nih.gov and looking in the /pub/image/demos directory.

Whilst every endeavour has been made to make MacStereology effective, the MacStereology program is supplied ‘as is’ and there is no guarantee that it will perform any particular function.  No liability for any consequences of this program, whether intentional or otherwise can be accepted.  Errors – Yes, like most Macintosh applications, there are some unexplained errors in the program.  Some may be due to memory fragmentation so restarting MacStereology will sort it out.  If it ‘quits unexpectedly’ suspect that the allocation of size is inadequate.  New additions to the program sometimes produce strange and unexpected effects elsewhere.  Please report errors (with as much detail as you can supply) so I can correct them.  Suggestions for improvements and your particular requirements will be welcome.

.c.Section 2 – Test data files

MacStereology is supplied with a range of data files to enable you to test the program.  These are data files for 3-D display and images that can be analysed.

.c.2.1	3-D test files

These files are designed to show you the sort of 3-D reconstructions; that are possible.  Use a copy of them so that the original settings are not lost by mistake.  Most of the options described (such as Wireframe, Layers and Surface) will be found in the 3-D menu.  For a Macintosh Plus or SE you should reduce the sizes of the plots (Magnification in Position dialogue of 3-D menu).  Since these must use bit map patterns, the plots will be clearer on these machines if you select Mark edges (Lighting dialogue).

If you lose the plot, just select Reset 3D (3-D menu) to get a reasonable (though not ideal) plot back.  Alternatively Close (File menu) the file and Don’t Save the changes then reOpen it.

2.1.1 “Pear” data file	(See Section 6.8 for illustrations)

Either start MacStereology by double clicking the application then Open (File menu) the data file, or just double click the file “Pear”.  The first frame of the file is displayed, which is just the bottom of the pear.  (If the digitising tablet is in use when you Open a file, the last frame will be displayed – tip of the stalk – since it will be assumed that you want to collect more data)   The dialogue shows how many frames and objects there are in this data file (pressing the mouse button down will stop it disappearing, but you can get the same information by calling Ster Help in Apple  menu).  Plot a 3-D reconstruction by selecting Layers.  The default shading is a different shade for each layer (shades of yellow and green if you have a colour Macintosh).  Try Order (3-D menu) to plot the layers in reverse order (click button labelled 14-1 then click OK) or any jumbled order (e.g. type 1-5 12*3 7-4).  Use the  Position (3-D menu) dialogue to change the Magnification (e.g. type 0.5 then click OK).  If you lose the pear you can always select Reset 3D to bring it back (in shades of red and yellow).  Use Order to restore the original order of 1–14.  It can be saved in what ever state you last displayed it, when you Close (File menu) the file.

Solid (3-D menu) takes a bit longer to plot.  You should now see that the pear has a bite out of it.  The Resolution of plot can be varied from the Position dialogue (3-D menu) – choose a higher value for Resolution for a faster plot at lower resolution.

Wireframe (3-D menu) will show a plot similar to how it would be sent to a digital plotter.  Note, that it is plotted from the top to the bottom, since the masking for a plotter must be precalculated.  The dotted parts of the reconstruction are calculated from the co-ordinates of the original data, so will line up if you make a stereo pair by using Viewpoint (3-D menu) by clicking Stereo pair with Separation of 62.5 mm and Angle 5°, but you will need to reduce the size of the display or increase the spacing to separate the two images.

When you have finished Close (File menu) this data file (Don’t Save changes) then Open (File menu) another or Quit (File menu) from the program.

2.1.2 “Square” data file	(to display a cube)

This file actually consists of just one object with only 4 points in just one frame!  Open this data file by selecting Layers (3-D menu).  If no file is open then any command that requires a file automatically calls Open first.  The resultant plot is a perspective view of a cube consisting of 11 layers (1*11).  By default this file has been set up as a stereo pair with the standard 62.5 mm spacing.  By selecting Order (3-D menu) you will see that the single frame is repeated 11 times.  If you select the button marked 1-1 you will then get a plot of just one layer.  Changing the Distance to 0 in Viewpoint (3-D menu) will change the plot to isometric.

Use Order to return to the full 11 frames then try the Wireframe plot.  Note that the lines of the visible part are interpolated up to the point where they disappear.  If the option from Select is for Lines + dots, this will look similar except the corner of the masked part appears as a single dot.

The Solid reconstruction plots the surfaces of the cube.  The plotting is quite rapid since there are only 4 surfaces to be established for each layer.  Try changing Fill in the Select (3-D menu) option.

When complete, Close the file but Don’t Save any changes.

2.1.3 “Sphere + Cube” data file

This shows a sphere floating inside a cube.  If you call Select (3-D menu) you will see that the plotting mode of the cube is Solid and not Filled sphere and the sphere is a Surface.  Try changing plotting modes and colours, e.g. the cube in Surface mode and not Opaque.

You can try the effect of changing the Viewpoint (3-D menu) to see how Rotation and Tilt change the appearance of the plot.  If you Tilt the reconstruction towards you but stop the plotting half way through by holding the mouse button down you can get a cut away view looking into the sphere (don’t Fill).  The same effect can be achieved by using Order (3-D menu) to plot only a few layers.

2.1.4 “3-Ply plait” data file

Each layer consists of 3 circles mathematically generated to make a plaited rope.  Use Layers to see the general form, but the shading/colour gives little idea of the structure.  The default scale is probably too large for your Macintosh screen, but plotting can be stopped by holding down the mouse button until the current layer has been plotted.  The Wireframe plot will start off the top of the screen and eventually produce a jumble of ellipses!

Choose Solid to see the detail.  The three strands of the plait are now obvious.  You can vary the amount of Depth shading from Lighting (3-D menu) – 0% entirely orientation to 100% entirely depth.  The size of the facets can be changed using Resolution in Lighting (3-D menu).  A value for Resolution of 1 or 0 will produce small facets showing a fairly smooth surface.  To get an even better surface use a Resolution of 3 then set Interpolate shading to 5 in Lighting.  A final change to remove the layer lines can be made by removing the filling for all IDs (Select dialogue, press the Command ⌘ key then click not Fill) and setting the Line thickness to zero in Lighting.

2.1.5 “Blood vessels” data file

This is a file of real data showing a small part of a network of branching blood vessels.  In Layers or Wireframe this does not look particularly interesting, but the Surface reconstruction shows the complexity of the branching, but if you want to remove the colours either use Mac config. (File menu) to show it in grey tones or Select (3-D menu) to make all Highlight and Shadow colours the same.

2.1.6 “Split” and “Split*” data files

These files contain the same data with some changes made to the Linkage (3-D menu).  They are designed to demonstrate two different ways of showing how objects can be made to split and join.  Plot these files to see the effects.  If the Split* file is shown in colour you can see how the joins are constructed.  The section on Linking objects (Section 5.7) shows how these files differ.

2.1.7 “bat_cochlea” data file

This file contains data from MRI images of the scala media of a moustached bat (Art Keating, University of North Carolina).  The stack of 115 images was analysed with 4 additional images added to define the split/join in the way shown in the “Split” data file.  In each the division of the 2 objects was defined by rubbing out a few pixels in the ‘neck’ between 2 features.  Use Order (3-D menu) to see how these were added to the data.

2.1.8 “Smooth & Points” data file

This shows an example of smooth surface rendering, translucency and point mode blobs.  The appearance of the points can be changed using Select (3-D menu).  Now find the ID of the points by typing a ‘3’.  Change to Colour by layer to make the points a uniform colour on each layer or Show point surface to give them a more solid appearance (the latter message becomes Fixed colours if the ID does not refer to an object in point mode).  To make a uniform colour on throughout all layers make the 2 values in Range equal.  Making the colours for Highlight and Shadow the same will also give uniform colour, but in this case the points will not be outlined.  Spherical points change the shape of a point from spheres (i.e. always plotted as a circle) to discs in the plane of the section that will plotted as ellipses according to the Tilt of the Viewpoint (3-D menu).

The other three objects have been set with interpolated shading.  The amount of interpolation can be changed in the Lighting (3-D menu); a value of 1 will not subdivide facets, whereas the maximum of 9 will divide each facet into 81 sub-facets.  You can also try interpolated surfaces with the other demo data files, but change the resolution to about double current value.

The blobs are can be seen through the other structures since these have be made not opaque.  This is controlled from the Select dialogue, with the degree of translucency from the Lighting dialogue.  Since the translucency effect is produced by alternating pixels of the images printing halftones will not work, but the display can be printed from the Print (File menu).

2.1.9 “5 cm” data file

This is a test file for calibrating your LaserWriter.  If you print this file you should get a set of exact 5cm squares.  If you need to change the scaling there are X- and Y-scale factors in the resource fork of MacStereology (STR# 400) which can be changed with a resource editor (ResEdit).

2.2	Analysing binary images

The folder “Images” contains a selection of Pict files that can be displayed or analysed. Images look better if centred on the screen that is the default in Read Opts (Image menu) but this does not alter the analysis.  By default files read by Read binary are automatically Analysed as soon as the file is read.  The analysis would normally require an appropriate Magnification (Options menu) or Calibration (Image menu), but this can be skipped if you just want to see the binary analysis in operation.  Most of the options used in analysing images will be found in the Image menu.

2.2.1 “2 Circles” Pict file

The two circles differ by a single pixel gap at the top of the right hand circle.  Analyse finds the circle to the left and an arc for the other, which is correctly interpreted as a line not an object with a small area with a perimeter equal to the line length (and not double this value that it would be if it measured the round the inner and outer surface).  Note that such lines are labelled at the mid point of the line, using text in outline mode with the user font.

2.2.2 “McD” Pict file

This is a simple MacDraw picture consisting of a square circle and curved line.  Shows the finding of profiles and the recognition that one is a line.  Try the effect of selecting Chunky pixels in Anal. opts and check the difference by using List data (Data menu)

2.2.3 “Nested.McD2” Pict file

This consists of 3 shapes with internal features.  The default analysis finds all the internal features, but only the outer margins would have been located if Find nested features had not been checked in Anal. opts. (Image menu).

My thanks to John Russ and the original authors for the following data files, which were originally grey-tone images, but are displayed as binary files in MacStereology.

2.2.4 “Cornea” Pict file

Change Font to something smaller e.g. 9 Pt Times (Data menu).  Label by object number (Options menu).  Analyse - 396 objects.  List data (Data menu) looks better if units are mm (Units menu).

2.2.5 “Dendrite” Pict file

You will need to Invert image from Read Opts (Image menu).  Analyse – 154 objects including many small particles.  Set minimum limit in Anal. opts. to 100 pixels reduces data to 79 particles.  Try other limits, e.g. Max of 1000 pixels excludes 20 largest objects.

2.2.6 “Diatom” Pict file

To avoid black square showing, read this file after checking Invert image in Read opts (Image menu), then Find nested features in Anal opts(Image menu).  This field contains many small features so avoid numbering by setting Label objects (Options menu) to None.  Remember to set the size limits in Anal opts back to 0 to 100,000 else you will not find anything!  There are 990 objects of which the first is the whole outer profile of the object.  Setting maximum size to 10,000 skips this first object and leaves the rest looking neat against a black background.

2.2.7 “Grains” Pict file

Invert the image and analyse.  It will find 324 objects.  Use  Anal opts to vary the minimum size between 100 and 1000 pixels.

2.2.8 “SEM particles” Pict file

This can be read as a grey tone image using Read image (Image menu) or as a binary image using Read binary.  To analyse use Read opts to Invert the image.  A simple analysis finds 74 objects, but includes many small blips that can be ignored by setting a Min. size in Anal opts (Image menu) to 50 pixels.

2.3	Analysing a sequence of Pict files.

.c.2.3.1	Automatic analysis of a sequence of Pict files.

The folder “Split.picts” contains a sequence of binary images (saved from the data file “Split”) which consists of the profiles of the 10 sections.  Read one of them using Read image (Image menu) if you want to see what they look like (either 1 or 2 blobs).  To analyse the complete sequence set up the options in the Read opts. to read a Numerical sequence.  Leave Centre image checked.  Click OK..   Use Anal. opts. to specify the details of the analysis by selecting Save coordinates, Analyse immediately, IDs in sequence and Average pairs of points (the blobs are quite large so this will halve the number of coordinates saved).  There are not any Nested features so it does not matter whether this box is checked.  Go to Read binary and read the first file “Split.PICT.1”.  The image will be read and analysed, then you will be prompted for Magnification (answer 1) and section Thickness (answer 1.5 unless using small screen when you should use 1), give the data a file name (e.g. Split**) and finally specify the Data format (enter 15 frames, 30 objects, 100 characters of text, and if you want a title).  After this the other images in the sequence will be read automatically and analysed.

Split.PICT.1	Split.PICT.5

You can now review the data, but it is probably best to get rid of the last image using Clear image (Image menu).  You can display individual section using Draw frame (Options menu) or scroll through them using the ‘<‘ and ‘>’ keys (there is no need to press the Shift key as ‘,’ and ‘.’ do the same).  Using ‘+’ (or ‘=‘) and ‘-’ you can go to the next frame and also call the Draw frame dialogue.

(N.B. It is NOT possible to save the data files on disk with the demonstration version of MacStereology, but “Split” and “Split*” contain similar data for you to see the results).

To display the reconstruction you should select Auto clear (Options menu or Lighting in 3-D menu) so that from now on the screen will be cleared before each reconstruction.  A Surface (3-D menu) reconstruction shows one structure going through all layers and a separate part looping off to the left (green on a colour monitor).  To make this part of the main structure use Linkage (3-D menu) and find Frame 3 (just type a “3”).  Click the number in the centre of the blob that is shown in outline in the window at the right (for frame 4) and it will be shaded showing it is now linked.  Go to Frame 8 and repeat the process with the unfilled object in the window at the left (frame 7).  Now click OK and the reconstruction will be replotted with the links shown.  If the colour is not wanted, use Mac config. (File menu) to turn Use colour off.  The facets are a bit large so reduce the value from 5 to 1 (or 0) in Resolution in Position dialogue (3-D menu) which will mean that instead of using every 5th point in the data set it will use more of them (up to a maximum of 127 facets per surface).  You could also move the position on the screen here, but it is probably easier to drag the display (press ‘Command’ ⌘ key if necessary).  If the refresh is being calculated, rather than using its normal off-screen image, dragging moves a dotted line corresponding to the area of the screen or window and not the image.  Remember that holding down the mouse button stops the display at the end of that layer.

The display should now look almost correct.  You may want to improve it by changing the following parameters:–

Viewpoint - Rotation to 0° and Tilt to 5° to see details of the split and join.

For users with a colour display it may be better to see the result in grey tones - turn off Use colour in the Mac config. (File menu).

Lighting - give it some depth shading, say 50%.

If you want to be able to see inside the display, use Select, then hold the ‘Command’ key down to remove the Fill for all IDs, and finally use Viewpoint to change the Tilt to about 45°.

Change Font and Font size if you wish and the Goodies.

Finally the display may look better on a Black background.

Close the file or Quit when you are complete and Save the data with the plotting parameters.  Add this file to your demo files “Split” and “Split*”.

.c.2.3.2	Analysis to show partially merging surfaces.

Normal sequences can be done using the simple analysis, but this is an example of a more complicated analysis.  The aim of this example is to produce a split and joint that is intermediate between the sharp cut of the data file “Split” and the broad merge of “Split*”.  For this the 10 Pict files are used plus modified images to define the split on sections 3 and 8.  These have 2 extra images (3a, 3b and 8a 8b) which were obtained by rubbing out part of the images to show the part of the profile that should link to the double features on the central layers (4-7).  Since these extra 2 files originated from the same full profile they will fit exactly to show an elegant split and join.  Contrast this method of doing splits and joins with those shown using the demo files “Split” and “Split*”.

From the Image menu set the options in Read Opts.  To analyse this sequence select Multiple files not Numerical sequence since we need to treat the extra images differently.

Set the options in Anal. Opts (Image menu) by checking Save coordinates, Analyse immediately, Show End frame dialogue, Identifiers in sequence and Average pairs of points. Select Read binary (Image

menu) and in the folder containing the demo images and Open the file “Split.PICT.1” (double click file name, but since it is top of the list and already highlighted you just need click Open or press the Return key).

The image file will be read (a black blob near the centre of the screen), and (since you are saving coordinates) you will immediately be prompted to Change magnification - Click OK or press Return.  Then enter section spacing (1.5 for normal Mac II monitor, 1 for a Mac Plus) and click OK or press Return.  Give the data a file name (e.g. Split***) and click OK or press Return.  Enter new data structure (e.g. 20 total frames, 50 total objects, 100 characters of text, and may be a new experiment title - “Split & Join”) then click OK or press Return.  It is always wise to set a generous limit for the number of sections (frames) and total objects within these frames.

The first image will be analysed (1 object) and the End frame dialogue will be displayed.  Just click OK or press Return.

The Read binary dialogue will then come back.  Select “Split.PICT.2” and repeat the process.

After reading and analysing “Split.PICT.3” click Cancel (NOT OK) on the End frame dialogue as we want to add more data to this frame.  Read the file “Split.PICT.3a”, note that it is analysed and displayed as a different colour/pattern since it is the second object in frame 3 its ID is 2 (which defaults to a green object).  Again click Cancel on the End frame dialogue and read and analyse the third profile from file “Split.PICT.3b”.  This completes frame 3 so this time click OK or press Return on the End frame dialogue.

Files 4, 5, 6 and 7 are analysed the same as file 1 (but 2 objects per frame).

Files 8, 8a and 8b follow the same route as files 3, 3a and 3b.

Files 9 and 10 are back to the same as file 1 (a single feature per frame).

After file “Split.PICT.10” and its End frame dialogue the data file is complete.  Click Cancel to the next Read binary request.

A 3-D reconstruction can be done now, but you should check Auto clear (Options menu) to clear the display automatically before each reconstruction.  Select Layers (3-D menu) and you will get the first sight of the shape of the structure.  The data needs some minor editing before doing the Surface plot, otherwise it will not link correctly.  This can be done either by changing IDs or changing links.  The following procedure is changing IDs using Edit data.

Select Edit data (Data menu) and find frame 3.  This shows the 2 objects that will form the join to the divided part of the structure (IDs 2 and 3) and the single object that links to object on frame 2.  However, on frames 4, 5, 6 and 7 it is necessary to change the ID of object 1 from 1 to 3.  The ID number is the value in the box to the right of arrows on the line showing ‘Object:-’.  As you change it the profile will be replotted (changing from orange to blue if you are using colour).  After these four changes click OK or press Return.

Select Surface(3-D menu) to see the full reconstruction and improve the display in the same way as in the previous example (Section 2.3.1).

Close the file or Quit when you are complete and Save both the data and the plotting parameters.  Add this file to your demo files “Split” and “Split*”.

2.3.3	Creating Pict files from a data file

Even if you are using a digitising tablet and not entering images from Pict files this type of procedure will provide a route of changing the data without having to redraw the profiles by hand.  Equally you can create your own Pict files using applications like SuperPaint and MacDraw II.  For the above analysis the Pict files were made from the data file “Split”.  Select (3-D menu) was used to change both Highlight and Shadow colours to black (pressing the ‘Command’ ⌘ key to change it for all IDs).  Numbers in the display were removed by setting Label objects (Options menu) to None and the images were saved as a sequence of Pict files (File menu).  Subsequent modification of these files was made using Image 1.29.

For reworking the data file “Blood vessel” the tall images needed to fit to the screen; to do this all the images were rotated by 90° using the Align (3-D menu) and their sizes reduced from the Tablet scale (Tablet menu) before saving them as Pict files.  Where the original data contains objects inside features it would be necessary to make the internal feature white, then analyse nested objects;  alternatively it is possible to save the different objects in separate files and analyse several files for a single layer as was done above for the “Split” file (Section 2.3.2).

.c.2.3.4	Other options for Reading and Analysing images

The Read opts and Anal opts dialogues in the Image menu controls how images are read and analysed.

If you find your images are half off the screen, suspect that they have been saved with an unusually large frame (e.g. the size of a full printed page).  In Read opts taking Centre image off will leave the image in a more sensible position.  This image analysis routine uses a copy of what is on the screen for the analysis and hence will not analyse any features that are off screen.

If you want to move images around before analysing them you will need to use Anal opts to turn off Analyse immediately.  Drag the images (or use the arrow keys plus the modifiers ‘Shift’, ‘Option’, ‘Command’ and ‘Control’).

The offsets will be shown in the Read opts dialogue and will be applied to future images if you check Offset.  Alternatively you can specify values of these in the Read opts dialogue.  The difference between the two methods is that the shifts specified in the Read opts dialogue will apply to all subsequent images (if Offset is checked), whereas moving an image does not apply to subsequent images.  The offsets required to centre an image will also be shown (after an image has been read) in the Read opts dialogue and can be made to apply to subsequent images.

The size of the image can be changed from Read opts using Scale and Scale to fit.  The result can look a bit ragged (also images from some applications are not displayed if magnified over x1.1 - reading the images into another package and resave should cure this).

The size of a binary image that can be analysed is limited to the size of your monitor screen;  any features off the display will be lost and any overlapping the edge will only be measured up to the edge.  If necessary, Scale the image to reduce its size.  If you are analysing many small objects use Label objects (Options menu) to remove the object number/ID or reduce Font size (Data menu).

Smoothing and Averaging of data is controlled from Anal opts.  For an object about an inch across the perimeter will have about 300 points (the screen is 75 pixels per inch).  This is more than enough to describe the shape and I would recommend that you reduce the data by averaging pairs of points (unless there is some fine detail that requires the full resolution).  Averaging data loses almost no useful detail, but this not only almost halves the size of the data files, it also speeds up subsequent displays.  Smoothing data applies the binomial coefficients shown to the data and modifies the data that is saved.  This may be useful in getting an accurate perimeter length, because it removes the zigzags as the boundary follows the pixels, however, data for 3-D reconstruction can also be smoothed as it is displayed using Goodies (3-D menu). The 50µm steps used for saving all co-ordinates corresponds to a scale factor of 7 shown in Tablet scale (Tablet menu), so averaging or smoothing data will not result in values rounded to the nearest pixel on screen.

If you intend to do a 3-D reconstruction you must select Save coordinates, but if you just want to measure the features (e.g. area or perimeter) you do not need the co-ordinates and this will greatly reduce the size of the data file.  Save coordinates appears in Anal opts (Image menu) and is the same as in I-O config (File menu).  Save coordinates can only be checked before collecting any data.

If you normally change several of these options you can reconfigure MacStereology permanently to your preferred options using Defaults (File menu) so that your settings come up each time MacStereology is started.  This sets a whole range of values so be sure that you do not have any strange values elsewhere before saving.  To avoid this problem, Quit then restart MacStereology and just set the options you want to make defaults.  Data files and templates also save these settings.

.c.2.4	Making a Movie

A movie consists of a sequence of images that are saved from the screen display.  Typically up to about 100 images can be used, but this is limited by the size of the image and the amount of the available memory.  The images may originate from a calculated reconstruction, Pict files or any other image that can be displayed.  Two of the standard movies can be done automatically – Movie (Image menu) for a sequence of Pict files and Sequence (Data menu) for an animated 3-D reconstruction. Alternatively individual reconstructions of images can be added to a movie or even images entered using Paste image (Image menu).  All images in a sequence will use the same colour tables so do not mix Movie with Sequence unless you have disabled Colour/grey tones in Mac config. (File menu).*

All images in a sequence are saved using the window size of the first image of the sequence.  When displaying the movie, you can change the window size.

Since full 640 x 512 screen with an 8-bit image occupies 300 kbytes each, you will rapidly run out of memory unless:–

a.	Use a small window for images by calling Window in File menu then dragging the size smaller using the grow box in the bottom right corner.

b.	Use a Mac with at least 4 Mbytes memory, then expand MacStereology from the default 1.5 Mbyte, or use Finder instead of MultiFinder if using System 6.

c.	Reduce resolution (4-bit colour is adequate for a simple reconstruction. Use Monitors in Control panel (Apple  menu).

d.	Restart MacStereology.  Some data structures are created permanently whilst the program is running so reducing or fragmenting available memory.  Use Ster help (Apple menu) to see the free memory, but this figure varies dynamically.  A good indication of low memory is when the menu bar is slow to refresh or the menus act sluggishly, but the only sign may be a the program quitting with no free memory or a system failure with an MultiFinder error #1.

e.	Set Recalculate screen refresh using Mac config. (File menu) to release the memory of the off-screen image.

.c.2.4.1	Muybridge photos; (photographed in about 1878 and republished on the cover of Scientific American October 1878).

Use Read image (Image menu) to read any image from sequence.  Position it tight into the top left by dragging or cancel Centre image in Read opts.  You can also Scale this image from Read opts.

Change to Window (File menu) and reduce window size tight on to image by dragging in bottom right corner (the drag box is not normally shown, but will invert as the cursor moves over it).

Select Movie (Image menu) and check Numerical sequence then OK.  Open the first file in sequence (“Horse.1”) and the complete sequence of 12 Pict files will be read and the animation will start.  (If you run out of memory in doing this the program will first remove the refresh buffer then if all else fails it will quit.)

Control the speed of animation with the number keys ‘1’ – ‘9’ (‘zero’ = stationary) or increase (‘+’ key) or decrease (‘–’ key) time between photos .  Letter ‘O’ changes to oscillate (i.e. images 1 to 12 then 12 to 1), ‘F’ for forward, ‘B’ for backwards.  Stop running by pressing mouse button or typing ‘.’.  Restart by using Run movie (Data menu).

Stop the movie by pressing the mouse button and try Fade in Data menu.  A value of 2 produces a smoother run by inserting an intermediate frame with alternate pixels from each image (try bigger fades to see how it works).  Fading is slow, so maximum speed will be reduced (‘1’, ‘2’ keys will probably do nothing unless the images are very small).

When finished, use Erase movie (Data menu) to clear out the sequence to get rid of the last display.

.c.2.4.2	Animated 3-D reconstructions

Open data file “Cube+Sphere”.  Use Position (3-D menu) to reduce Magnification to 0.5 and change Resolution to 2.  Make a Layers reconstruction.  Drag display to top left corner.  Use Goodies (3-D menu) to remove annotation.  Change to Window (File menu), and reduce window size fairly tight on the display.  Now use Sequence (Data menu) to make the animation.  Click the text of Frames in sequence to find maximum possible.  If this number is too small for the animation reduce monitor resolution, make window smaller or remove refresh buffer  by checking  Recalc. refresh in Mac. config. (File menu).   Use a value a few less than the maximum to avoid memory problems.  If maximum was 80 frames, try 72 - set Rotation 0 to 90° (1.25° steps if 72 frames) and Tilt from 0 to 360° (5° steps).  Animation will look better if you avoid the zero angles so try Tilt 2.5 to 362.5° and Rotation 0.6 to 90.6°.  A Layers plot is faster, but Surface gives a more interesting animation.  You can follow progress of the reconstruction from the count in the menu bar.  Note that in this case a Rotation of only 90° works because of the symmetry, but the full rotation of the Tilt is required since the light sources move with the reconstruction; 15° is not the same as 180+15=195°.  The lighting is directed as though from the equatorial plane and hence tilts with the display;  examine a tilting sphere to see how this works.

An alternative display is to remove the cube by setting it to Don’t plot in Select (3-D menu) then just show just the sphere.  If you then use Linkage (3-D menu) to remove the caps of the sphere (Click Caps off in top left corner) and Select to remove Fill from the sphere (ID=2) then the animation will let you see inside the truncated sphere as it rotates.

You can add different sequences together (e.g. a changing Rotation then a changing Tilt, then add additional images with features in different plotting modes, but not colours since all images in a sequence use the same colour tables).  An existing sequence can sometimes be recoloured, without having to be replotted by using Select (3-D menu) to change the colours, but this only works with simple reconstructions that would have similar colour tables.

Erase movie (Data menu) when you have finished with it.  Close (File menu) the file or click the go away box in the top left corner of the window, but don’t save any changes.

.c.2.4.3	Animation to show stages of reconstruction

Use “Cube+Sphere” data file at about 0.6 magnification in a Window as before.  Plot the Surface then Start movie for the first frame.  Use Select to change plotting modes - click Fill,  replot and then Add to movie (beeps to confirm OK).  Add a few more frames, e.g. Cube as Surface, Surface but not Fill, Dots, then change the Sphere (ID=2) to Solid.  Run complete movie, but slow it down using a value of between 8 and 32 using the Fade option (Data menu).

All parts of a movie use the same colour tables, so make sure no colour combination is omitted (Don’t plot in Select) from any image.  If you totally remove the cube the sphere is plotted in its correct colours, but in the movie will appear in shades of red not blue since it was created with a different colour lookup table.  In doing a Sequence or Movie the colour tables of the last image are applied to the whole sequence, so you may expect strange results if these change.  You may be able to use Select to change the colours then display an existing Sequence immediately without having to recalculate.

.c.Section 3 – Testing the digitising tablet

.c.3.1	Setting up the tablet

Plug the digitising tablet into the appropriate port (serial port or ADB port) and, if necessary, turn on the power.  Start MacStereology and from the File menu go to IO Config dialogue.  Select the appropriate tablet input by specifying whether there is a serial tablet that is plugged into the printer or modem port or an ADB tablet that is plugged into any ADB port (back of computer or keyboard);  there is no need to check .i.Save co-ordinates; for testing.  On clicking OK the program will attempt to hand shake with the tablet and you will get messages confirming that this is correct or that some error has occurred.  If you get an error, check the tablet is plugged in correctly and powered up, or cancel and then use .i.IO Config again; to confirm that you have selected the correct input.*

Current ADB tablets should use the standard interface released in 1990 and be automatically configured, provided the tablet has been installed (drag copies will not work, even though the tablet acts as a mouse).  If you are using the old Summagraphics Bit Pad Plus digitising tablet you may need to remove the tablet driver from the system file and Restart (Special menu from the desktop) – consult the user’s handbook for details.

.c.3.2	Using the tablet

Assuming the tablet has established its hand shake you can now draw shapes on the digitising tablet with the cursor.  MacStereology will only record co-ordinates when the Read button is pressed (the button nearest the end or pressing the pen tip on to the tablet).  You should see the track of the cursor drawn on the Macintosh screen.  The default display size may be smaller than the tablet area, so if you want to move the display area or use the full area of the tablet you should change the .i.Tablet scale (Tablet menu);.  It does not matter if the trace goes off the edge of the screen, but you must stay within the active area of the digitising tablet.  If .i.Auto clear (Options menu); is selected the screen will be cleared before each object, otherwise you can draw objects on top of one another.

MacStereology ends an object when either it detects that you have drawn the line back to the starting point or you specifically send a termination command.  The shaded area around the start of the line on the screen indicates the limit that is used when you are closing the object.  When the track of the cursor re-enters this area the program will start checking for the closest approach to the starting point and will only then stop data collection.  The values for the area and .i.perimeter will be displayed at the top screen.  The other 3 buttons on the cursor can be used, whilst you are drawing the profile of an object, to control the data collection.  You can Cancel the data and erase the path (or from keyboard with X), finish data collection and force a Closed object (C) or leave an .i.Open; line (O), in which case the object will have a perimeter length but cannot have an area.

If you stop pressing the Read button, it stops recording data and waits until you continue.  If you move to a new spot then push the button, it will draw a straight line to the new position.  A square can therefore be drawn by clicking the corners.  To avoid any problems in drawing the final point back at the origin it is best to force the object Closed rather than trying to put an exact point at the origin.  If you close by scratching around near the origin the ‘square’ may finish up with 10 points of which only the 5 describing the square are useful.  This may not matter to the appearance of a single layer, but it can have an unfortunate effect on a Surface plot since this assumes the points round an object are uniformly spaced.

.c..c.3.3	Using a microscope with a digitising tablet

The best way to digitise a specimen, which can be directly viewed under an optical microscope, is to use a drawing tube attachment with the microscope and to view the cursor on the tablet by mounting a small lamp over the cross hairs.  By reducing the brightness of the microscope lamp it is possible to see the bright spot of the lamp in the field of view.  Curiously such a device is not a standard attachment for digitising tablets, but a simple one can be made using a miniature l.e.d. (I normally use a red one) run from a 9v battery (with a series resistor to give the correct current) fixed over the cross hairs with some sticky tape.

Some electron microscopes also have a

drawing tube attachment.  The lamp needs to be much brighter so that the spot of light can project through the drawing tube attachment onto the phosphor screen.

3.4	Using the data

If you didn’t select Save coordinates in IO-Config (File menu) you won’t be saving the co-ordinates in a file.  After the first object it is not possible to save co-ordinates, but you can still save a summary of the data including all the values shown if you .i.List Data (Data menu).  Data can be output as text files or images.

Try using the .i.Units; menu or changing the .i.Precision (Data menu) and list the data again to see the effects of such changes.  All data is saved at full precision and as absolute values that can be scaled to any units.  By default the measurements are in cm at a Magnification of x1.  .i.Magnification (Options menu); applies to ALL objects in a frame, so if you are measuring objects at a different Magnification you should first use .i.End frame (Options menu); to finish the current frame and start a new one.  The value of the Magnification is the scaling between the real sample and the image on the screen (normal Macintosh screen is 72 or 75 pixels per inch).  It is also possible to use Calibrate to measure the magnification.

.c.Section 4 – Using MacStereology;

There are three main ways in which you may want to make measurements:–

a.	A series of single measurements of area or .i.perimeter; of single objects (e.g. areas of a selection of peaks in a graph, or lengths of DNA molecules).  Normally set .i.Auto clear (Options menu); so that the graphics for last object are erased as the next profile starts.

b.	Measurements of features on a series of .i.frames;, where each frame is probably a different micrograph.  The data could then be used for stereology.

c.	Recording features on a series of frames, in order to manipulate the co-ordinates corresponding to these object (e.g. serial section reconstruction).

In the first two options the data may or may not be saved on disk, but in the third option the full data, including the co-ordinates, must be saved.  To save data on disk or save changes made to existing data the file must not be locked and the disk must not be write-protected.  Simple display of existing data can be made with locked files or a write-protected disk.

All the derived parameters (area, perimeter, centre of gravity, etc.) for each object are stored in memory, together with the co-ordinates corresponding to the last object measured.  The data is divided into a series of frames.  A frame just indicates some sub-division of the data.  Some parameters are global for the whole data set, but some, such as magnification and thickness, are applied to the objects in a frame.  Most commonly each frame is a separate sample or micrograph, so that the values for individual objects measured on that frame can be used to calculate the average properties for that frame (e.g. for stereology) or one section for a 3-D reconstruction.

.c.4.1	Data Collection using a digitising tablet

There are several ways that features can be recorded from the tablet.  The profile of an object can be drawn to measure areas (a Closed object), or a simple line (an Open object) or a series of points (Point mode).

.c.4.1.1	Using the cursor buttons

The buttons on the cursor control the flow and use of data whilst the object is being drawn on the digitising tablet, and not at any other time.

The position, colour and meaning of the 4 buttons vary between models - experiment to establish which of the buttons is ‘Read’, ‘Cancel’, ‘Open object’ and ‘Closed object’.  Experiment to find how your tablet works.

With the .i.MacTablet; or MM1201-type, .i.GTCO and Kurta; tablets the 4 buttons are coloured.  Only the Read co-ordinates button (pink) is coloured on the .i.Bit Pad Plus; cursor:–

| 4-button | Bit Pad | Keyboard | Function |
| --- | --- | --- | --- |

| Yellow | Pink |  | Read co-ordinates |
| --- | --- | --- | --- |

| White or Red | White | X | Cancel set of co-ordinates currently being measured |
| --- | --- | --- | --- |

| Blue | White | O | Finish object– Open object |
| --- | --- | --- | --- |

| Green | White | C | Finish object– Force closed |
| --- | --- | --- | --- |

The cordless mouse used by the Kurta tablet is particularly convenient to use.  It is also possible to use a cursor with only a single button or the stylus.  In this case the only command from the cursor/stylus may be Read and the other functions are made from the keyboard with the letters ‘X’, ‘O’ and ‘C’.

An .i.open object; (line) is terminated by pressing the Open button on the cursor or typing letter ‘O’.  This results in a curved line that has a ‘perimeter’, i.e. it has length, but has no area.  The Closed button on the cursor or letter ‘C’ makes a line into a closed object by drawing a straight line from the last co-ordinate back to the first point.  If the Cancel button is pressed whilst measuring an object, the co-ordinates corresponding to that object are rejected and the profile will be erased from the screen.

.c.4.1.2	Drawing an object

The profile of an object is drawn using the hand cursor on the tablet and the corresponding (x,y) co-ordinates are recorded at approximately regular intervals around the object that can be adjusted using .i.Step size (Options menu);, Frame ID or Obj ID (Tablet menu).  To define an object accurately it is necessary to have sufficient data (probably at least 50 points), but having too many points is inefficient and may even reduce the accuracy of some measurements, particularly the .i.perimeter; (because every wobble of the hand will be added into the calculation making the resultant value too large).  Note that the scale/magnification of a drawing can make a considerable difference to the perimeter, e.g. the coastline of Britain would be much longer if measured from all the 1 inch to a mile maps than if it is measured using a pocket atlas.  Typically a step size of 1 mm is used, although for very small objects a smaller step size may be required.

The normal limit is 0.05 mm (50 µm) since a value of 0.0 mm will result in data being collected at a rate that is dependent on the speed of movement of the cursor.  Each object ID may have its own step size.

The program collects co-ordinates while the Read button is pressed.  On releasing this button, collection stops and the program waits until it is pressed again.  To draw an object with straight edges the cursor can be pressed once at each vertex.  An object may consist of up to 4096 points, but typically it will be only 100–200 co-ordinates.  Whilst recording points the menus are dimmed.

The object being drawn may be Closed or Open.  A .i.closed shape; is automatically recognised when the cursor gets back to the starting point.  The display will be filled with a pattern that depends on the object identifying number (change using .i.Select (3-D menu) ;in 3-D menu).  When you are drawing an object, the first point of the display is ringed by a circle of shading (actually 8 times the step size).  This indicates the distance that the cursor must move before the program starts to check for a closed object, and then is used to locate the closure.  The program does this by noting when the cursor re-enters this circle, then continuing to record data until it detects that the cursor is starting to move away from the start point.  The previous point, i.e. the point of closest approach, is then used in subsequent calculations.  Although the closure is recognised automatically, it is sometimes better to force closed from a point just before the return to the starting point.  An open object is just a line and hence obviously cannot have an area.

During data collection the mouse cursor displayed on the Macintosh screen is changed to a picture of the 4-button cursor and the menus are dimmed.  It is not possible to use the mouse to access the menu options whilst you are in the middle of drawing an object.  At other times the normal Macintosh mouse can be used to access the menus, but the cursor from an ADB digitising tablet will change back to a normal mouse as it moves into the menu bar.

Almost as soon as one object is finished the computer is ready to start the next object.  To avoid it immediately starting another object, before you have time to take your finger off the Read button, there is a delay that can be changed using the .i.Tablet Wait; (Tablet menu) from the 0.25 sec default.  If you are too slow and the next object starts, you will see the circular target at the end of the last object and a straight line to the current object.  Press the Cancel button to remove this incorrect data.

Between leaving the circle at the start of a line and re-entering it, any shape may be drawn, including crossing lines, but be aware of the effect that this may have on the calculation of area (a symmetrical figure of 8 will have almost zero area since the area is calculated with a sign that depends on the direction of drawing).  Only the absolute value of the area is saved, so the direction of drawing is normally irrelevant.  You can make use of this effect to subtract a hole from an object, providing you draw both as parts of one object.

Whilst you are drawing on the tablet, the incoming stream of co-ordinates is continually recorded and checked to see whether they are valid and appropriate.  Those that are wrongly spaced or in some other way erroneous (e.g. illegal character in the text) are rejected.  The valid co-ordinates are displayed and stored.  For a serial line digitising tablet, if there is any spare time during drawing, the derived values are calculated, otherwise the calculations are completed after the object has been drawn.  For an ADB tablet all calculations are done as the co-ordinates are received, but this is not likely to slow data input except on a Macintosh SE with .i.Show X-Y (Tablet menu);.

The derived parameters are .i.area;, .i.perimeter;, .i.centre of gravity; and the .i.limits of x and y;.  All these are stored in tablet co-ordinates (steps of 50 µm) and will be scaled by the .i.Magnification; or .i.Scale ;and converted to the specified Units (default x1, measured in cm).  As soon as the object is completed the circular target is removed and the object may be filled in (see .i.Fill objects; in Options menu) using the Colour/Pattern according to its ID that can be chosen using .i.Select ;(3-D menu) or set up by default.  Finally the graphics are annotated with the object number or object ID as specified by .i.Label objects; (Option menu).  If the co-ordinates are being saved these will then be written to disk.

.c.4.2	Point mode

Data from a tablet can be input one point at a time and a blob displayed for each point*.  Use the Cursor/Pen to pick off all points of one type.  When in Point mode (Tablet menu) and the tablet is active, there is a warning in the menu bar.  All similar blobs count as a single object and will be displayed with a circle on screen, whose size can be selected by the user.  You can use several IDs for different sizes and colours (e.g. nuclei, mitochondria, vesicles, etc.).  Record a series of similar points then complete the group of blobs by either a Close or Open buttons (both have the same effect) or using ‘C’ or ‘O’ keys or Stop in the Point mode menu.  Cancel (by button or ‘X’ key) deletes one point at a time and not the whole sequence of points.

The diameter of the blobs displayed can be specified from the Object ID or Frame ID (Options menu) or Point diam/Step size (Tablet menu) options.  The text in these items changes to reflect whether you are using point mode.  During data collection the same variable is used for step size and blob size.  In the data file each set of points has its own point diameter.

The blob diameter is saved as part of the data for each object (group of blobs), so need not be the same for all objects of the same ID.  The blob diameter can also subsequently be changed using Edit data (Data menu) and any profile can be switched between Closed, Open and Point mode (this recalculates the derived values).  The Area and Perimeter for blobs are the total for all blobs in that set using the specified diameter.  As with all other objects, blobs are plotted using the specified Magnification and Units, but stored in the same way as coordinates.

In 3-D reconstructions the points are scaled to size and can retain their circular shape (i.e. assumes that points are spherical) or shown tilted (i.e. assumes their shape is a flattened disc).  Blobs can be shaded to give them a more solid appearance.  Note that all blobs within one object have the same size, colour and shading.

Although an object ID would normally refer only to a single type of feature it is possible, but not recommended, to specify a closed object, an open line and a series of points using the same ID.  Volumes (Data menu) will warn you of this.

When in Point mode there is a ‘beep’ as a point is collected.  These beeps can be disabled using Beep points (Tablet menu).  This option also has an effect when drawing a normal object.  When a Closed object is completed the computer beeps and there is then a delay to give the you time to take your finger off the button (Tablet delay in Tablet menu) before it is ready to start the next object, when it then beeps again.  Beep points can be used to stop this second beep.

.c.4.3	Identifiers for objects and frames

Each object has an identification (ID) number and may also have a name.  The Object ID (Options menu) can be used to change the ID.  This number is used to classify objects in any subsequent analysis or display.  Any value in the range 1 to 255 is possible and the name (any length, but normally up to about 20 characters to fit the box), e.g. on a blood film ID=1 may by ‘RBCs’, 2 ‘Neutrophils’, 3 ‘Eosinophils’, etc., with ID=20 for ‘extraneous material’ and 10 for the ‘reference frame’.  New identifiers can be added at any time.  The names of existing identifiers can also be changed;  the new name will apply to any previous data with that identification number.  A data file of these names can be saved for future use using Save template (File menu).

The names are purely for your convenience and need have no particular significance.  When using a digitising tablet to input data, each object identifier may also have its own .i.Step size;.  The step size indicates the approximate distance (in mm) between points on the tablet.  The data collection routine takes measurements or saves co-ordinates each time the pen moves this distance on the tablet.  This procedure has several advantages – the number of data points does not depend on the speed of movement of the cursor, small wobbles of the hand are not recorded as separate points and the points used in a 3-D reconstruction are evenly spaced.  The Step size (Tablet menu) is normally just the global step size, but it can be changed if, for instance, the objects are very small or very large.  When that object ID is selected the step size will be changed to the value that was specified when that ID was last used.

Note that objects are automatically numbered 1, 2, 3… in a frame, but the object identifier can be any number (1–255) and in any order with repeats or omissions.  The display on the screen can be left unlabelled, labelled with the object number or with the ID number using Label objects (Options menu).

During data collection there are a number of short cuts for changing the object ID.

a.	Use IDs in order (Tablet menu) or in Anal opts (Image menu) to let the program change the ID after each object.  By default the ID will be the same as the object number, but this can be changed before the object is measured, or afterwards using Edit data (Data menu).

b.	Increase or decrease the ID using the ‘<’ and ‘>’ keys (or ‘,’ and ‘.’).

c.	Increase or decrease the ID as well as showing the dialogue using the ‘+’ (or ‘=’) and ‘–’ keys.

d.	It is not necessary to use the menu or press I to get the Object ID dialogue.  Just type the new value (the normal dialogue will be shown after the first digit is typed) then press Return or click OK, e.g. ‘2’, ‘5’, ‘Return’ for ID=25.

When the ID is changed it will recall the details of the definition of that ID when it was last used.  This includes whether it is point mode and the step size or diameter.

The Object ID dialogue changes depending on whether you are using point mode.  The dialogue changes if Point mode is clicked:–

Each frame also has an identification number and may also have a name that can be set from Frame IDs (Option menu).  Just as with objects, the number is used to classify frames for subsequent analyses.  The only difference from object identifiers is the step size box, which in this case just operates on the global step size (the same as calling Step

size).  Frame IDs can be automatically incremented to match the frame number;  this is controlled from End frame (Options menu) or Anal opts (Image menu).  This can be used to write a comment for each frame (e.g. Frame 1, ID=1, name=‘Blood film, Slide 5’).  Since each frame is normally a separate print it may also be necessary to specify its .i.Magnification; and Thickness (if required for subsequent 3-D reconstruction or in some stereological calculations).

.c.4.4	Correcting errors

Although it is best to avoid errors in your data it is possible to correct most mistakes at a later stage.

a.	If possible, correct the errors as you measure the data:–	
If the current object has an error whilst you are drawing it, just press the cancel button to erase it and redraw the object (in pont mode cancel removes one point at a time)

If it has the wrong identifier (ID is displayed at the bottom left of screen), change it from the Option menu before drawing it.  After it the object is completed use .i.Edit data (Data menu); to correct it.

b.	If the last object is wrong, delete it with .i.Cancel ;(or X) then draw it again.  A common fault when drawing a very small or an irregular object is that the drawing passes through the ‘home’ circle before you have finished drawing the object and hence cuts off part of the object.  You can often avoid this by starting at the furthest extreme of the object, but if this fails it may be necessary to change the .i.Step size; (Tablet menu).

c.	If an earlier object is wrong its profile cannot easily be modified, although its identifier can be changed.  It may be possible to use .i.Delete object or Delete frame (Options menu); to delete data until you delete the error.  Delete object when there are no objects left in a frame will also enable you to reopen an earlier frame.

An alternative way to lose an object is by changing its identifier zero.  When collecting data, IDs must be in the range 1–255, but .i.Edit data (Data menu); can be used to make ID=0.  That data is then ignored in all displays, although it still exists in the data file and can later be changed back to a valid ID.

d.	It is also possible to create a new version of a complete frame.  Reject the data for the old frame by changing its Frame identifier, if data is being classified for stereology using the frame IDs, or using the .i.Order (3-D menu); option for 3-D reconstruction, e.g. if frame 5 of 25 frames is wrong and it is redrawn as frame  26, then order would be 1–4 26 6–25.

e.	Measured values can be changed from Edit data (Data menu).

.c.4.5	Input from Binary (Pict) files

By using either a .i.video ;camera and a .i.frame grabber; or a flat-bed scanner; it is possible to save images from serial sections as a sequence of Pict files.  Binary read (Image menu) lets MacStereology read these files and convert the profiles into the boundaries that it uses for 3–D reconstruction.  The Macintosh expects the images for analysis to be .i.binary images; (i.e. 1-bit or black and white only) with the features of interest as black blobs or lines against a white background.  Binary read will read grey-tone or colour images but display them as binary images.  Read opts (Image menu) allows images to be inverted, shifted or scaled.  Anal opts (Image menu) allows .i.nested objects can be found      to be found, reject small/large objects and recognise objects that are .i.;lines rather than closed objects.  If the images have been saved as a numerical sequence, the whole set of images can be processed completely automatically.

The binary Pict files should be prepared using your own image processing application.  There is a free package available called Image that can capture images from a Data Translation QuickCapture or Image Systems Technology (Scion) Image Capture.  Do whatever image processing you need and then segment it to a binary image (e.g. by thresholding the grey-tone image) and maybe massage the binary image to remove spurious features or add missing pixels.  If you save this image as a Pict file with an incrementing numerical suffix (e.g. File1, File2, etc.) MacStereology can automatically read and process the complete sequence.  The numbers of the files can make any sensible sequence (File would be treated as File0, extra digits may appear as in File9 then File10 or if you prefer File09 then File10, and the sequence need not start at number 1).

Try to save the sequence of images with the correct orientation for a subsequent 3-D reconstruction, but the Read opts (Image menu) can shift a binary image before it is analysed and both rotation and alignment can be changed after they are digitised using Align (3-D menu).  After analysing the first image, the second file is displayed and it will be overlaid by the measured boundaries from the first section.  Providing Analyse immediately in Anal opts (Image menu) is turned off, the binary image can be moved around by .i.dragging; with the .i.mouse;, pressing .i.⌘; if this was selected in Mac config (File menu).  Alternatively the image can be nudged using the .i.arrow keys (←↑↓→); to move in single pixel steps or x2 if Shift key pressed, x4 with Option key, x8 with Command (⌘) key or x16 with Control key (only present on newer keyboards).  Once the .i.position; is correct, call Analyse (Image menu).

Select the appropriate options in the Read opt and Anal. opt dialogues.  These may include Single Pict file, Multiple Pict files or Numerical sequences, Analyse immediately each image, Show End frame dialogue, Invert image (black⇔white reversal) and Finding nested objects.  The object identifiers can be assigned in sequence (up to 250) as it finds objects.  The resultant data file may need some identifiers corrected, but this is easily done using Edit data (Data menu).  Alternatively the Object ID dialogue (as called from Options menu) can be displayed for each object as it is found;  this has the advantage that you can use names with each identifier, but it tends to be slow.  Each object can be given same identifier, but this is not appropriate if you want to do a 3-D reconstruction.  Finally the IDs can be found automatically from the previous frame using those objects in the same position.  Use Label objects (Data menu) to change how these identifiers are displayed.  If you want to use a .i.frame identifier; this should be entered before analysing the image if the End frame (Options menu) is not being displayed.

Each image is scanned across each line from left to right starting from the top of the image and working to the bottom looking for the first pixel of each object it finds.  During analysis the menus are dimmed and the cursor hidden.  The analysis of the rest of a frame is skipped if the mouse button is held down.  By default, when the analysis is complete the frame will be Ended and the next analysis will be a separate frame.  You may, however, have several images for each layer (e.g. as in “Split” data example described in section 2.3.2) and want them to be part of the same frame for reconstruction;  for this you should either show the .i.End frame; dialogue (set from Anal. options) after each image then Cancel it if you want to run on the next image or use Delete object (Options menu) to remove the end frame marker after each image has been analysed.

Pixels are considered connected if there is another pixel adjacent or diagonal to it.  .i.Single pixels; are ignored and a size discriminator can be used to reject small or large objects.  A very large object may be too large since no object may have more than 4096 points, e.g. a square of about 1000 x 1000 pixels, but the limit may be reached with a much smaller object if the boundary is irregular.  The boundaries of objects are coloured as they are found and filled if Fill objects (Options menu) has been selected.  The profile is displayed and numbered according to the option selected in the Label objects (Options menu).  For small objects either omit the number or use a small Font (Data menu).

Errors can be corrected by using Delete object or Delete frame (Options menu) and reanalysing it, using Edit (Data menu) or frame .i.Order (3-D menu); to skip a frame or substitute another.

The data is saved as the co-ordinates of the pixels of the boundaries.  To match other data a scale factor of x7 is used to make the steps 50µm on the screen, which is the factor shown in the Tablet scale (Tablet menu).  Anal opts (Image menu) enables the data to be Smooth;ed or reduced to half its size by Averaging pairs of points.  If the data is smoothed the perimeter is calculated from the smoothed data (area is not affected by smoothing).  Remember that boundaries can also be similarly smoothed during 3-D display using the .i.Goodies; dialogue (3-D menu).

Where objects divide or join it is possible to merge the sections or to define a division (see section 2.1.6).  Simple merging features requires to specify the Links.  To define the split as in the bat_cochlea data needs extra images to repeat some layers with the precise division drawn.  These are added either before or after the full image.  To make them plot as a single layer the dividied section should have zero thickness if it is after the full section, whereas the full section should have zero thickness if it is before the divided image.  Examine the “bat_cochlea” data file to see how this is implemented.

.c.4.6	Measurement and Stereology

In doing a stereological analysis the protocol adopted will depend on the analysis required.  Particularly where the data is to be analysed statistically or if using stereological formulæ, which are based on precise statistical theory, it is essential to design the experiment properly.  Consider whether a random sample is required or whether a more efficient systematic sample is possible.  Within each frame establish how many objects need to be measured and how they should be selected.  Problems may arise from objects that overlap the edge of the frame since they may be fully or partially included or excluded according to the analysis – use the identifier to separate the various groups.  In contrast to data for a 3-D reconstruction, it is usual to use the same identifier for all similar objects, since average values are calculated.

For each object the measured co-ordinates are used to calculate various parameters.  .i.Area; and .i.perimeter; that can be used to calculate volumes (VV = AA) and .i.surface area;s (SV = 4/π BA for an isotropic solid) by applying the appropriate stereological formulæ.  Other parameters are also calculated – centre of gravity and limits of x and y.  Previous versions of this stereology program calculated orientation, degree of orientation, maximum diameter (and its orientation), Ferret diameters, length and breadth of the object, although I have never had cause to use these and hence have omitted them from MacStereology.

For further information about new stereological tools see the review by H. J. G. Gunderson and co-workers in Acta Path MicroBiol Immunol Scand 1988 96 379-394 & 857-881.

Section 5 – 3-D Reconstruction

.c.5.1	Formatting data

3-D reconstructions are calculated from data obtained from serial sections through an object.  Each section (of known thickness) will be a frame in the data file.  You must save the co-ordinates describing the profile of each objects.  If you are doing a complex reconstruction, think carefully about the organisation of the data before you start digitising the data.

To start a new analysis it is probably best to Quit and then restart MacStereology to ensure all variables are returned to their default values.  You may read in a template containing object ID names and other details.  Specify .i.Save co-ordinates; from .i.IO config.; (File menu) or Anal opts (Image menu) before you record the first object.  If not already specified you will be prompted after the first object to enter the Magnification (Options menu), Thickness (Option menu), file name, c.f. New (File menu) and Data format (Data menu).

The data limits shown in Data format cannot be changed after the first object has been saved.  Decide what is the maximum number of .i.frames; (sections or layers) you will be using, but allow a few extra frames for safety and the wasted space from empty frames is not large.  The maximum number is 255 frames.

To establish the maximum number of objects that you will measure, multiply the number of objects per section by the number of sections and allow a safety margin, e.g. 4 objects per layer, 30 layers is 120 objects so enter 150 or 200 as .i.maximum objects;.  The maximum number of objects is 32,767, but this may be limited by free memory.

If you are using names for frames or objects, estimate the total number of characters in all these names and allow adequate space in the Total text.  The User name and .i.Experiment title; are stored separately from this space and can be changed at any time.

.c.5.2	Positioning the sections

.c.5.2.1	Input from Binary (Pict) files

By default a Pict file will be placed with its frame in the centre of the screen.  Using Read opts (Image menu) you can change this to offset it by a specified shift and/or scale the image.  If parts of the image are outside the screen they are lost and will not be analysed.  After the image has been read a copy of it is saved internally.  Subsequent movement using the .i.arrow keys (←↑↓→); or dragging the image uses this saved image so features going off-screen are not permanently lost.  You should try to record these images with the correct orientation and alignment so that MacStereology can read them without needing to adjust their position.  If reference lines or planes were in the specimen block exact registration should be possible.  However, it is possible to adjust the alignment of sections both as they are being digitised and with the final data file using Align (3-D menu).

.c.5.2.2	Using a .i.digitising tablet

Work out where your complete sequence of layers will come on the digitising tablet.   It is not unusual for a stack to ‘wander’ away from the first layer so make sure it does not go outside the active area of the tablet.  If you have reference lines or planes going through the object it will greatly help alignment.  The .i.Tablet scale; (Tablet menu) for the graphics can be adjusted for size or position at any stage through the project, however the Print/.i.Plot; option from the .i.End frame; (Options menu) uses the origin from Tablet scale to position the output on the paper.

The prints to be digitised can be placed directly on the tablet.  To stop them sliding around I normally stick a clear acetate sheet on the tablet so that the prints can be slid underneath (and you can trace key reference positions and your sections from the print on to the acetate for aligning sections).  For specimens that can be viewed down a .i.microscope; it is best to use a drawing tube attachment with the microscope then to mount a small lamp or l.e.d. over the cross hairs of the cursor (see diagram in Section 3.3).  By adjusting the brightness of the specimen it is possible to see the dot of light in the field of view as you look down the microscope.  I have used this both with my Wild optical microscope (a 10cm high stand to raise the microscope makes the field of view on the tablet larger for accurate measurement) and with a Zeiss EM109 electron microscope where the intense lamp actually projects onto the phosphor screen.

Draw the profiles in the first layer then finish it with .i.End frame; and Print or Plot it.  Take this drawing and fix it to the surface of the tablet at exactly the place where it was digitised as a reference for the alignment of subsequent layers.  For a microscope with a drawing tube this can be done directly, but with prints it may be necessary to print on to tracing paper or an acetate film so it can be hinged over your prints.  It is essential that the first drawing or print is in the correct position in order to make subsequent ones correctly align with this one.  For maximum accuracy align the print with the image in the microscope.  It should be the same position as lining up the edges of the paper, but small errors in position of the paper tend to be cumulative. The exact size of .i.LaserWriter; output can be changed by modifying the X– and Y–scale factors in the resource fork STR# 400.  If you do not have access to a printer you could just draw the profiles onto a piece of paper.

Another approach is to trace each layer onto a sheet of paper then stick the first layer onto the digitising tablet.  Subsequent layers are then adjusted for best fit and stuck on top.

The display of a single frame can be dragged around the screen using the .i.mouse;.  Click and hold the mouse to move the image*.  You may also need to press the Command ⌘ key if that option was specified in the Mac config (File menu).  The co-ordinates have been saved so the display can be redrawn on releasing the mouse.  If you are in the middle of recording serial sections and printing out the frames to help with alignment, you should not move the display nor change its scale.  Movement does not invalidate the data, but just shifts the plot on the next printout.

Thickness will use the units you have chosen from the Units menu. This indicates the distance between digitised layers (not necessarily the section thickness if you digitising only selected sections, i.e. if you are using every third section with 5µm sections enter a thickness of 15µm).  Choose an appropriate value, so that you minimise the number of sections that you need to digitise, but don’t lose detail.  If there is a lot of detail in part of the sequence it may be possible to reduce the distance between layers.  The object identifiers will enable the program to keep track of objects, even though their position varies considerably between layers.

.c.5.3	Identifier numbers

Either assign reference numbers (and names) to the features in the sections before you start or add them to the list as you go along.  The general principle is to number each feature in a section with a different number, then use the same number for that feature on subsequent layers.  If an object divides, introduce a new number for the fragments (carry on with one numbered with the original number if you want).  Never have two features with the same identifier if you intend to use the .i.Surface 3-D; display option.  For the analysis of Pict files let the program assign the IDs of the objects in sequence then, if necessary, change some later using Edit (Data menu) or do the assignment automatically using Get IDs from last frame in Anal opts (Image menu).

You can use Save template (File menu) .i.to make a dummy data file;, if you will be repeating the 3-D reconstruction for several samples, by saving the data file after specifying all the variables and names or from an existing data file.  Open this file then immediately use Save as to start a new file for the actual data.

If you are going to massage your data by using the .i.Align ;(3-D menu) you may also want to digitise additional reference points or lines that will not be plotted in your 3-D reconstruction.

For .i.stereological ;measurements you may need to record the complete frame (e.g. the rectangle of the print or an area within the print) and you may have to note which objects are overlapping this area by giving them a different identifier.

Where features are inside other features, such as a cylinder with an inner and outer wall, it is best to record the outer feature first, then the inner feature(s).  When you plot this using the .i.Layers; option you will find that the inner feature will punch a hole through the shading of the outer feature, rather than the outer feature obliterating the inner feature.  This will be done automatically when analysing .i.Nested; features from Anal. options (Image menu).

.c.5.4	Subsequent sections.

All variables defined for the first section are carried forward for subsequent sections.  When an ID is recalled all the details such as Step size/Point diameter and Point mode are recalled.  If necessary, change the .i.Thickness ;– there is no need to make the layers uniform in thickness.  For 3-D reconstruction do not change Magnification between layers – be careful not to nudge the zoom control on the drawing tube if you are using a .i.microscope;.

Input of binary Pict files procedure is quite simple.  Digitise the first section then input the next file.  This will be displayed (black and white) together with the boundaries (colours, .i.grey tones; or patterns) found on the previous section.  The new section can then be aligned by dragging it (using .i.⌘; key if necessary) or using the .i.arrow keys;.  It should also have the correct orientation compared with previous sections.

With a digitising tablet the print for the next section can be fixed on top of the first one - if you do NOT change the .i.Tablet scale; you will find that these prints will fit exactly on the first drawing.  Since I have no proper reference markers in most of my specimens I use these prints to align data by eye.  I find it useful to fix these drawings along one edge, so that they can be hinged, to compare alignment over many layers.  Final adjustment of the alignment (X– and Y– shift) and rotation can also be made later using .i.Align (3-D menu);, but it is much better to get it correct before digitising.

The recording of data can be stopped at any stage.  When you restart the program (or double click the data file) you will be returned to the position at which you finished, but you will need to use IO config (File menu) for a digitising tablet.  The only point to remember is to leave the drawings of the previous layers fixed on the tablet so you can align the new sections.  It is not important if the microscope has been moved;  just adjust the image of the last section to overlap the drawing on the tablet.  However, make sure the zoom control on the drawing tube, objective lens and the height of the microscope above the tablet are the same.

.c.5.5	Displaying a 3-D Reconstruction

The size and position of the .i.plot; are the same for all three plotting modes, although you will notice that the .i.Wireframe ;plot is from top to bottom, whereas .i.Layers ;and .i.Surface ;go from bottom to top.  The display is on the same scale for the Mac screen, the printers or the digital .i.plotter;.  Show page (3-D menu) shows how your plot fits onto the different size pages according to the selected Output (3-D menu).  To design a page for printing use the Layers option for rapid plotting, select Output and indicate the device then .i.Show page; to preview the page.

This preview always uses the Layers reconstruction, but it can then be printed in any of the three types of reconstruction.  A complicated Surface plot on the .i.LaserWriter; will take some time to compose and print, so a .i.print spooler; is recommended (see section on ‘Printing’ below).  Plotting or Printing can also be done by selecting .i.Output ;then calling WireFrame, Layers or Surface.  Reconstructions can also be Printed directly from the File menu.

Normally you will want to clear the screen before displaying a 3-D reconstruction so select ..i.Auto clear (Options menu) or from Lighting (3-D menu);.  To plot a .i.Stereo pair; use the .i.Viewpoint; (3-D menu) – an angle difference of 5° and a spacing of 6.25 cm is the standard view for stereo pairs but will result in a rather small display on the screen although this can be viewed with a stereo glasses.  For better quality make a larger display and photograph.  Stereo pairs of slides can be projected using twin projectors and polarising glasses.  For this I normally use an angle difference of 3°.

| Option | Wireframe | Layers | Surface |  |
| --- | --- | --- | --- | --- |

| Don’t plot | OK | OK | OK | Object not plotted, nor masks other objects |
| --- | --- | --- | --- | --- |

| Dot/Dash | OK | OK | OK | Co-ordinates plotted, other objects masked |
| --- | --- | --- | --- | --- |

| Line+dot/dash | OK | OK§ | OK | Masked part shown dotted |
| --- | --- | --- | --- | --- |

| Solid | OK | OK | OK | Masked part hidden |
| --- | --- | --- | --- | --- |

| Don’t mask | OK | OK§ | OK | Layers & Surface will still mask shading |
| --- | --- | --- | --- | --- |

| Surface | Solid | Solid | OK | WireFrame & Layers plots as ‘solid’ |
| --- | --- | --- | --- | --- |

| Colour Mac | OK | OK | OK | Colour or .i.grey tones |
| --- | --- | --- | --- | --- |

| B/W Mac | OK | OK | OK | Black & white patterns |
| --- | --- | --- | --- | --- |

| ImageWriter | OK | OK | OK | c.f. B/W Mac |
| --- | --- | --- | --- | --- |

| LaserWriter | OK | OK | OK | Half tone prints & fractional line widths |
| --- | --- | --- | --- | --- |

| Digital plotter | OK | (No) | (No) | Layers & Surface show the unmasked outlines |
| --- | --- | --- | --- | --- |

| Pict file | OK | OK | OK | Palette of indexed colours saved |
| --- | --- | --- | --- | --- |

| PostScript file | OK | OK | OK | Half tone prints & fractional line widths |
| --- | --- | --- | --- | --- |

§	These modes will take two passes to display.  Not recommended unless either not filling or the filling is white.

For a Surface plot up to 200 surfaces between any pair of layers can be plotted with a total of several thousands of facets per layer depending on the memory allocation (40 surfaces and 1500 facets should fit into 1 Mbyte with the standard 8-bit monitor, but for a complicated display you may need to expand the memory allocation of MacStereology).  Use .i.Linkage (3-D menu) tospecify whether an object ends as an open cut or has a cap added (shown as a cone of half the section thickness).  Select (3-D menu) will make a cut end with the Filled profiles display a flat end, but if unfilled will enable you to look down inside the structure.

The line widths used in plotting on screen or with a .i.printer; are determined using the .i.Pen size (Options menu); or from Lighting (3-D menu).  Zero pen width will remove the line from a shaded area, but not make lines disappear.  Fractional line widths can be used if printing on a LaserWriter or by way of a .i.PostScript ;file.

To stop the calculation of a display, hold down the button of the .i.mouse; until the current layer has finished.

To stop printing from the type command period (⌘ .).

The position of a 3-D reconstruction can be dragged around the screen using the mouse.  Click and hold the mouse, to move the image* .  Press command key .⌘ if that option was selected in Mac config. (File menu).  On releasing the mouse the display will be redrawn.  When holding the mouse button down to stop a 3-D reconstruction, to avoid the plot being accidentally moved, the button must be released before the display can be repositioning.

Perspective Surface 3-D reconstructions at low tilt angles (near 0° or 180°) can show the facets in the wrong order if the plane of view intersects the reconstruction.  A similar problem may arise when features are very far from aligning between sections, although the program does try to guess the best order of plotting.  Blobs plotted in 3-D reconstructions may be partly masked by the next layer at low tilt angles.  Masking in Wireframe is done by a very simple algorithm (looking at the highest y-value) so can show dotted parts that should not be masked.  On a digital plotter, masking is only done in Wireframe mode, but it is possible to plot Layers, Surfaces and blobs in point mode.

.c.5.6	Rotating and Tilting a reconstruction

Values for Rotation and Tilt can be entered from Position (3-D menu).  3-D reconstructions can also be rotated or tilted using the ‘rose’ in the bottom left corner.  Press the ⌘ key if necessary, and move the mouse into the corner.  The cursor will change and if the mouse button is pressed, the corner inverts and you can Rotate ( cursor) or Tilt ( cursor shown if within ±10 pixels of centre line) by dragging in a circle about the centre of the rose.  The angles are also displayed.  Whilst the cursor is within the inverted region you can press or release the button without losing control.  The screen will refresh only on leaving this region when the button is not pressed.

Reconstructions are now based on the average centre of gravity of all objects.  The resultant reconstruction will be near the centre of the screen if X– and Y–offsets are zero in Position (3-D menu).

The algorithms for surface display have been modified to cope with any Rotation or Tilt angle and to improve the appearance of animated sequences.  The light source is effectively two sources at opposite sides of the reconstruction.  The light and shade correspond to the light source being a strip at infinite distance.  To avoid too much change as a sequence of reconstructions is tilted, the angle of the light moves with the tilt.  Thus a tilt of 0° and 180° will not look the same.

.c.5.7	Linking objects; for complex surfaces

By default a .i.3-D surface; is drawn between objects with the same ID number on adjacent layers.  There is therefore normally no need to call .i.Linkage (3-D menu); before plotting.  The Linkage routine is used only where you want to specify features in complex structures such as joins, splits or caps at the end of a structure.  For a structure containing many objects it is preferable to change the IDs using Edit data to enable it to use the default links.  Even with complex structures, matching IDs can frequently be assigned using automatic options in Edit data (Data menu).

The Linkage dialogue looks complicated, but can be considered in three sections.  In the centre is the picture of the current frame (9) with the current object shaded (no. 4 with an ID of 3).  On the left is the last frame (8) and on the right the next frame (10) in the sequence.  Objects on the current frame may link with objects on the last or next frame.  The last frame corresponds to the section lower in the stack, whereas the next frame corresponds to the frame above the current layer.

Objects in the pictures can be numbered by their object number, by their ID number (as above) or without numbers.  Numbering can also be changed from Label objects (Options menu).  In general, displaying IDs is easiest to use, but showing object numbers enables you to view objects in the same order as you recorded them.  As you move the cursor over the three windows objects will be found and indicated by highlighting the number and changing the cursor from a hand to a cross.  Click on this in the centre window to select an object, and click in the side windows to change the links.  The display can be dragged using the mouse when it showing the hand cursor.  The size and initial position of the display are taken from .i.Tablet scale (Tablet menu);.

Scroll through the frames to find the one you want to link.  Then select the object using the mouse in the central display by moving to the centre (over the number if there is one) and clicking.  It is also possible to select objects either by scrolling through the object numbers (1, 2, 3… ) or the corresponding object IDs (values in order, but only those IDs actually used in the current frame; if you type an impossible ID number it will find the nearest valid ID).  Linked objects are shown shaded  You can select or deselect objects on adjacent layers by clicking objects in the other two windows or caps by clicking the Caps buttons.  Caps are shown by a cross in the adjacent frame (as can be seen in the top right of frame 10)  To break a link any number not an ID number in use can be entered in the ID box on either side, but clicking or using the Break link button puts the ID for at 255.

The four buttons at each side at the bottom control all the links to the next frame.  These can make or break all links to that frame, or add or remove all caps.  The four buttons in the top left of the dialogue do similar actions for the whole data file.  You will be warned before any of these actions is carried out.  The Default link makes links with object having the same IDs on adjacent layers.  If there is no object with that ID then no surface will be drawn.  If the link is to an ID=0, or Cap is checked then a conical cap whose height is half the section thickness will be drawn.

Linking works by associating two flags with each object to indicate which ID it links to on the previous frame and the next frame.  Similarly the objects on the adjacent frames have two flags each so they too can make links.  This redundancy makes possible almost any combination of links to make branching and joining object.  In practice clicking objects should make and break links as desired, but potentially this simple system of flags can forget some links, so suspect this if the resultant reconstruction misses a link.  Multiple links, including three-way or greater splits, can be built up by choosing the appropriate values of links.  For simple splits or joins the links can be specified in either direction, e.g. using either the pointer from the current layer back or the point from the previous layer forward;  it makes no difference which way is chosen.

.c.5.7.1	Changing links to interconvert data files Split and Split*

The two test files Split and Split* are based on the same data and can be used to demonstrate the effects of different sorts of linkage.  Normally a data file is not designed to be displayed in both modes, so the conversion between the two displays is a bit laborious.

By default a Surface display of Split* shows that the split and join merge.  Choose Linkage (3-D menu) and select Default links, then replot.  There are now two gaps in the structure.  The gap is caused by a change of IDs from 1 at the top and bottom to 2 and 3 in the centre divided parts;  by default objects only link to the same ID.  There are also two extra objects (IDs 4 and 5) on both frames 3 and 9 that can be plotted.  Use .i.Select (3-D menu); to change the plotting mode of these IDs to Surface.  Re-enter Linkage and on frame 3 make ID 4 link forward to ID 2 and ID 5 link forward to ID 3 on frame 4.  Similarly on frame 9 make IDs 4 and 5 link back to 2 and 3 on frame 8.  Now redisplay – the split and join are defined by the extra profiles.  This is now the same as the file Split.

To return the file to the original merged split and join either Close the file (without saving changes) then reopen a fresh copy, else using Select make IDs 4 and 5 Don’t plot then with Linkage find frame 4  and make both ID 2 and 3  link with ID 1 on frame 3 and finally on frame 8 make links from both IDs 2 and 3 with ID 1 on frame 9, then replot.

Note from the Order  (3-D menu) that frame 5 is being ignored.

.c.5.8	Printing 3-D reconstructions on a LaserWriter and saving Postscript files

Output of drawings from a Macintosh to a LaserWriter often uses bit patterns in making a print, so its quality is little better than an ImageWriter print.  The new print drivers with systems 6.0.7 and 7 can handle grey tones in some applications.  By addressing the LaserWriter directly using .i.PostScript;, the sequence of 64 .i.grey tones; (0=black to 63=white) used by MacStereology can be printed as .i.half-tone .i.shade;s; (similar to a photograph in a newspaper).  Half-tone shading is actually the default mode on a LaserWriter, which may explain why bit-pattern printing is so slow.  Note that the LaserWriter II SC cannot accept PostScript commands.  Some applications, such as PhotoShop can convert a grey-tone image from MacStereology into a dither dot pattern at LaserWriter resolution.

Printing can be stopped using the .i.Command-period; keys (⌘ .).

Printing from the Macintosh to an ImageWriter or LaserWriter can be very slow, particularly for a complex drawing, such as the .i.Surface ;presentation.  The delays are part the Macintosh system and are not within MacStereology.  The basic way for the Macintosh prints is to spool the graphics or text then set up a temporary file.  These operations require considerable space, both in memory (can be over a megabyte) and on your system disk.  Thus using a LaserWriter to print a bit-pattern image of the Surface from the ‘Pear’ data file originally took about an hour, hence MacStereology now addresses the LaserWriter directly in Postscript and takes only a few minutes to produce a print.

.i.Bit pattern; – .i.ImageWriter ;output

0							63

.i.Half-tone; – LaserWriter output

0							63

Because of the fine resolution on the LaserWriter all the specified line widths are halved, so the default is 0.5 pixel.  Line sizes from .i.Pen size ;(Options menu) or Lighting (3-D menu) can be fractional, although on the Macintosh screen they will be taken as 1 or more whole pixel widths.  The frequency of the .i.half tone; grid can be changed using .i.File options (File menu); from the default 75 lines per inch.  For the normal LaserWriter with 300 lines per inch, the default blob would be 4 x 4 pixels so relatively few .i.grey tones; could be used.  By changing the frequency to 37.5 lines/inch each blob will be 8 x 8 pixels enabling many more grey tones to be printed, but at a lower resolution.  A photocopier can then be used to reduce the size of the resultant print.  The specified half-tone frequency is saved in the data file.  A value of 0 leaves the half-tone frequency at its default.

If you intend normally to use a value other than 75 lines per inch you can change the default from the .i.Defaults (File menu).

An option in .i.Output ;(3-D menu) enables you to produce PostScript files.  These text files may be several hundred kbytes in size, but they can be easily and quickly printed on a LaserWriter.  Preparing and printing a separate PostScript file is slightly faster than printing directly on the LaserWriter.  You can use an application that can include PostScript files (e.g. MacAuthor/LaserAuthor).  Alternatively, it is possible to print them directly on a LaserWriter using a PostScript downloader (e.g. Adobe FontDownLoader) or even MacTerminal with the LaserWriter connected directly (not by way of AppleTalk).  You can also edit the file since it is simple text, but any error will stop the file printing.  Each PostScript file needs appropriate commands at the start and end.  These are included according to the File

Options (File menu):– PostScript command headings starting %!… and %%…, ‘gsave’ at start and  ‘grestore’ at end and ‘showpage’ at end.   MacAuthor/LaserAuthor requires the ‘gsave’ and ‘grestore’, but the default is for use with a FontDownLoader so files finish with ‘showpage’.  The scale factor in the plot enables plotting of the graphs  at higher resolution than the normal pixel size on the Macintosh screen (e.g. the default value 4 creates a PostScript command ‘0.25 0.25 scale’ so uses the LaserWriter at full resolution).

.i.Goodies (3-D menu); contains .i.smoothing options for the profiles of an object when plotted as a 3-D reconstruction in any of the three modes and for the grey tones of the display used in the Surface option.  For output of surfaces a value for Smooth shading of 4 is useful to make the graduation of grey tones smoother, but normally leave at zero if you have specified Interpolated shading in Lighting (3-D menu) else the shading may look rather flat.

For best detail of the surface choose a small value for the .i.Resolution; (from Position dialogue in 3-D menu), although this will slow the plotting.  For maximum resolution choose a value of 0 that will give each object 127 facets on each layer.

I  have a few suggestions which help printing:–

a.	Give MacStereology adequate memory.

b.	Use a .i.print spooler;, e.g. Print monitor or SuperLaserSpool™.

c.	Keep the graphics as simple as possible, especially avoid a .i.Surface; plot at high resolution or more interpolated facets than is essential.  The high resolution necessary on screen or for a slide maker is not required when printing since the resolution is lower.

d.	Use a Macintosh with a hard disk – more room for the spool file and faster.

e.	Use a Macintosh II and one of the newer LaserWriters.  Make sure you are using the latest system software (The original system 7.0 software should be upgraded for efficient printing).

f.	If the Mackintosh fails to print it should put up an alert message, but sometimes no error is detected and the program just returns to the main menu having apparently done nothing – simplify the drawing and try again.  Print monitor may indicate a specific PostScript error.

g.	If using system 7.0 make sure the Tune-up (1.1.1) has been installed.

Printing a reconstruction containing translucent features is likely to conflict with the half tone resolution.  This is therefore disabled and you will need to print the screen image from Print (File menu).  Remember, the screen size is different from the paper size.  PostScript files are also disabled.

.c.Section 6 – Reference section;

This section deals with all the items in the menus.  Use this mainly for reference since the use of the menus for the various types of image analysis has been described in the previous sections.

.c.6.1	Apple menu – 

There are two additions to the desk accessories installed in your usual list.

.i.About .Ster ( menu)

About Ster	The introductory dialogue includes the version number and date of .i.MacStereology ;and name and address of the registered .i.user;.  A new version number generally indicates some change in data structure which MacStereology needs to recognise when reading an old data file  All old data files will work with this version of MacStereology;  the largest change that may be apparent is in the positioning of the graphics on screen.  Data files created or modified by this version of MacStereology can probably be used with old versions, but there is no guarantee that they will be correctly displayed.

If this dialogue indicates that your copy is a .i.demonstration version; you will have the full working application except you cannot open a new data file nor delete data in an existing file.

.i.Ster help ( menu).i.Help

Ster help	Information about the current data, indicating how much data has been collected and the data file or sequence of images.  The current free memory is also list, but this varies dynamically.  It can be sent away by clicking the mouse button or pressing any key.

This same dialogue is displayed transiently after opening a file or deleting an object or frame.  To stop it disappearing, hold down the mouse button.

Control panel	Monitors can be used to change the screen resolution.  256-Colours is the normal setting, but reducing the resolution to 16 or 4 tones releases memory and may be necessary for Movie (Image menu) or Sequence (Data menu).  16-Greys is normally adequate if not using colour, although the basic range for each colour scheme is 64-tones.  If the resolution is too low for a full range of tones the display will switch to dither patterns using the basic 8 colours, then if all else fails to black and white.  If you subsequently increase the resolution you will need to re-enable Use colour/grey tones in Mac Config (File menu).  32-Bit QuickDraw is not normally necessary to give the full range of colour tones and it uses much more memory and may be much slower (e.g. with 8-24 video card).  After changing the screen resolution, the display will be recalculated.

Speaker volume can be used to vary the sound.  Apart from a warning in alert dialogues, sound is used when using a digitising tablet to indicate the end of objects and when it is ready to start again and for points.  Beep points (Tablet menu) disables these last two.

Keyboard can be used to change Key repeat rate and Delay until repeat that are used to control the frequency of scrolling in dialogues.

.c.6.2	File menu

This menu controls the production, reading and printing of data files and the configuration of your Macintosh and its devices.

.i.New (File menu)

New	Start a new data file on disk.  Old data, if any, will be saved first.

.i.Open (File menu)

Open	Open an existing .i.data file; from a disk.  The data file restores many of the options (such as magnification, whether to save co-ordinates and parameters for the 3-D display) to whatever you set them to last time you saved this file.  The total number of frames and objects recorded is then displayed.  If the co-ordinates of the objects were recorded it will draw a frame – the last frame if the digitising tablet is active (i.e. you are about to collect more data in this file) otherwise the first frame in the file.

.i.Close (File menu)

Close	Close data file and save any new data.

.i.Save (File menu)

Save	Save current data.  When collecting data, this is done automatically after each frame *

.i.Save as (File menu)

Save as	Save data as new file, leaving the old data file unchanged.  However, when adding data to an existing data file, the new coordinates are always saved (and the full details of the data are saved after each frame), even if Don’t Save is subsequent selected you would need to specifically delete this extra data, using Delete object or Delete frame (Options menu), before Closing.

It is also possible to Duplicate (⌘D) a file from the desktop, then make the modifications the copy of the data.

.i.Save template File menu)

Save template This creates a small file based on the current data (whether saved or not) which contains the information about the data including object names, colours and plotting details – essentially a dummy data file with no frames and no objects.  This file acts as a ‘stationary pad’ (its icon shows the ‘pages’) from which a copy is taken when it is Opened or double clicked.  On opening, this pad is unchanged but all the details are copied to memory ready to start data collection from a tablet or binary files.

.i.Text file (File menu)

TEXT file	Save data as an ASCII text file for other Macintosh applications (e.g. statistics).  The values are sent as a standard text file using a tab or return character to separate the fields.  You can either

make a file of the co-ordinates of a single object or of a summary of frames.  To avoid possible problems, numbers in text files do not use the thousands separators.

To save the co-ordinates for a particular object select the frame and object number.  Each object on each layer will be a separate file.  You can choose whether to transmit the number of co-ordinates as the first row of data in this file.  The data can be sent as integers as received from the tablet (i.e. steps of 50µm on the tablet).  Alternatively you can Scale the data according to your specified .i.Magnification (Options menu); and .i.Units; and using decimals numbers with the number of significant figures selected in .i.Precision (Data menu);.

A summary of the data consists of the values for each object from a range of frames and may include any of the items that you have checked.  Each row in the text file will include all the parameters you have checked in the order shown in the dialogue window.  Data can be scaled by your specified magnification and units and is sent using the number of significant figures specified.  If Headings is checked then each column in the text file will have a title.

The Application pop-up menu puts the appropriate file icon to the text file (so you can double click the data file), but does not format the text for that application.  [The selection of files and signatures can be changed using .i.ResEdit; on MENU 53 and STR# 401].*

The summary by ID presents the data as in Volumes (Data menu)

.i.PICT file (File menu)

Pict file	The pictures are generated the same size as you see them displayed on screen.  They use the screen size, but can be forced to use the page size from Page setup (default A4 if not called) and hence may be too tall for some graphics packages.  Use .i.Page setup; to change from portrait to landscape view or select other sizes of paper.

This routine saves the vectors for an object or frame, with or without Shading (c.f. MacDraw).  Use Tablet scale (Tablet menu) to change size of the picture.  (Scale size=7 is same size as on the tablet, size=1 will transmit the co-ordinates at full resolution).  The size of this dialogue depends on your monitor size.  The images can be dragged around using the mouse and this does not affect the position of the image as it is saved.

By selecting Pict file in the .i.Output; option (3-D menu) it is possible to make a file from a complete 3-D reconstruction.

Save Image/selection (Image menu) and Copy image/ selection (Image menu) also makes Pict files but these are the pixels displayed on screen, not the co-ordinates and graphics commands that go to make the display.  The difference is obvious if you greatly enlarge a small part of a saved image.

.i.Page setup (File menu)

Page setup	Called automatically before printer is first accessed.

.i.Print .(File menu)

Print	Has a sub-menu which enables various graphics to be printed:–

Data	Print the names of object identifiers and the .i.numerical values; of the data.

Frame	Prints the graphics of the object of the current frame.

3D Wireframe Prints the .i.Wireframe; reconstruction (see 3-D menu).

3D Layers	Prints the .i.Layers reconstruction (see 3-D menu).

3D Surface	Prints the .i.Surface reconstruction (see 3-D menu).

Image or	
Selection

Prints any display on the screen centred on the page, but loses the higher resolution of PostScript printing.

If not already specified these will call Page setup before printing.

IO-config.	Configure the serial ports for use with .i.printer;, .i.digitising tablet; and .i.plotter;.  This can be displayed at the start of the program to configure the .i.serial; ports and to indicate whether to .i.Save co-ordinates; for each object.  It is only possible to save co-ordinates if this box is checked BEFORE any data is collected, or a file that is already saving co-ordinates has been opened.  If .i.AppleTalk; is active, only the modem port will be available for the other devices.  It is not possible to select .i.ADB tablet; if you are running MacStereology on a Macintosh Plus.

If you select data input from the digitising tablet, it should be connected and powered up before clicking OK.  The tablet is configured by transmitting a stream of characters to it, and it will reply to the Macintosh.  The program will warn you at this stage if the Macintosh has failed to receive the acknowledgement from the tablet.  Selecting .i.Reset (Options menu); will also reconfigure the tablet.

.i.IO-config. (File menu)

Show IO config at start up and Make tablet active at start up are saved as part of the defaults in MacStereology and will be acted on next time MacStereology is started.

.i.Mac config. (File menu)

Mac config.	This option is mainly of relevance to users of a .i.Macintosh II;.  Normally there is no need for the screen refresh to be recalculated after using a dialogue box or desk accessory since the complete picture is saved in memory (about 300k for the normal 8-bit colour Macintosh screen but much more with a larger colour screen or 24-bit colour).  If the application starts running short of memory, there may not be sufficient room for this buffer and it will automatically switch to the recalculate mode.

You can select whether to automatically recalculate when you change a parameter that affects the 3-D display, rather than requiring a specific call to Wireframe, Layers or Surface.  If several parameters need to be changed, the full refresh after each stage could take too much time, but you can stop the calculation by holding the mouse button down.

Colour display can use the 4-, 8-, 16- or 32-bit video card, and can display the reconstructions in grey tones or colours (.i.Reset 3D; will include colours for each ID if Colour has been selected).

On a colour Macintosh you can show profiles or 3-D reconstructions in colour or black and white.  If .i.Colour ;is not selected, then colours will be converted into black and white or shades of grey.  This means that a file containing colour can be displayed on a monochrome terminal without adjusting any of the colours.  Note that the colours are numbered from 0 (black) with addition of red (1), green (2) and blue (4) to give other .i.shade;s (e.g. yellow = red + green  = 3).  Output to a LaserWriter, ImageWriter or PostScript file ignores colour information.

.i.Press ⌘ to drag graphics; is the normal mode since the mouse is normally used to define a region on the screen that can be used with Copy selection or Save selection.  Dragging graphics in dialogues never requires the use of the Command key.

.i.File options (File menu)

File options	You can select whether data files are saved automatically or after confirmation (during data collection the files are saved automatically after each frame and also after each object if co-ordinates are also being saved).  The way changes to the 3-D display can also be saved  can be varied.

This dialogue also controls the options for .i.PostScript files; (see Output in 3-D menu).

The Half tone frequency and Scale factor are used both for a PostScript file and in directly printing on a LaserWriter.  The default frequency is 75 dots/inch but you can specify a lower number to more .i.grey tones; or a higher number to show fine detail (see section on Printing on a LaserWriter in Section 5.8).  You are unlikely to use Scale factor. It indicates the difference in resolution between the Macintosh screen (75 dots/in) and the LaserWriter (300 dots/in), and allows integer arithmetic to maintain sufficient precision.

.i.Defaults (File menu)

Defaults	Modifies or reads the MacStereology start-up defaults.  To use, it is best to restart MacStereology, set the parameters you want saved (e.g. Font, Units or Binary size limits), then call Defaults and click select Change.

.i.Window display (File menu)

.i.Full screen (File menu)

This dialogue can also be used to Recall the defaults.  This will not alter the plotting parameters that can be reset using Reset 3-D, Default Links and Reset Align.

Window	Switches the display between the Full screen and a Window that can be moved or resized.  Note that the display will need to be recalculated if a new window is larger than the one in which the picture was originally drawn.  The window display enables access to the desk top and waste basket/trash can under Multifinder.  The full screen is used for analysing binary images.  List data (Data  menu) can also be carried out within a window, except that in this case the ‘go away’ box in the top left corner returns the user to the previous display.

.i.Quit (File menu)

Quit	Exit from MacStereology   If new data has been collected or old data changed you will be asked whether you want to save the data or changes before quitting.  You will also be warned to save measurements.
When collecting data from a digitising tablet it is automatically saved after each frame, so do not be surprised if there is no message about saving data when you quit; it has already been saved.

.c.6.3	Image menu

This menu controls the reading and analysis .i.binary ;.i.Pict; files

.i.Copy image (Image menu)

Copy image or

Copy selection  Puts current display (B/W, grey tones or colour) into Clipboard for transfer to Scrapbook or other applications.  This is saved with its palette, but not all applications can read this indexed colour information.  A region can be selected with the mouse, in which case this item changes to Copy selection.

.i.Pase image (Image menu)

Paste image	Enters image from other applications into MacStereology using the options set by Read opts.

.i.Clear image (Image menu)

Clear image	Clears the image that has been read in from a Pict file (by way of Read image or Read binary) and releases the memory used by its off-screen image.  If a data file containing co-ordinates is open any other graphics will be refreshed.  Note, that Clear image complements Clear screen (Options menu) which only clears the graphics and will leave any image.

.i.Save image (Image menu)

Save image or

Save selection  Creates a Pict file of the current display using the current colour tables.  For saving colour/grey tone images this give the best results and the images can then be edited using NIH Image, PhotoShop, SuperPaint or similar applications.  This contrasts with Pict files from Pict file (File menu) or Output (3-D menu) which contain vectors for the lines and shaded areas.  A region can be selected with the mouse, in which case this item changes to Save selection.

.i.Read binary (Image menu)

.i.Read (Image menu)

Read image	Reads any Pict file and will display it as a binary, grey tone or colour image.  This uses the system palette, rather than any special palette defined by the file or MacStereology, so dither of some colours may be noticeable. Full colour/grey tone display of digitised data is therefore also disabled.  32-Bit colour is possible if you install the 32-Bit QuickDraw Init in your system folder, but 32-bit colour graphics are slow using the 8-24 video card.

Reading a grey-tone image with the monitor set to 16-grey-tones may use only a limited number of grey tones.

Read binary	Reads any Pict file but always displays it as a binary image and then, if Analyse immediately is checked, executes whatever options are specified in Anal opts

A binary (1-bit, i.e. black and white) image can be saved in Pict format from a frame grabber, image analysis application or even SuperPaint or MacDraw.  These files use the options selected in Read options.  The image can be d.i.ragged; with the .i.mouse; (press .i.⌘; if that option was selected in Mac config).  Alternatively the image can be nudged using the .i.arrow keys (←↑↓→); to move single steps or x2 if the Shift key pressed, x4 with Option key, x8 with Command (⌘) key or x16 with Control key.  If a previous frame is displayed then the binary image will move behind this display.

Since analysis requires the full screen the window mode (Window in File menu) will be disabled so that the image will always use the full screen.

.i.Read options (Image menu)

Read opts	Controls reading of images for Read image and Read binary.  Images can be scaled, positioned or inverted.

The image may be Scaled to fit the screen (retaining the original aspect ratio) or Scaled by a specified factor.  Unless the scale is a multiple/division of 2 the image may be slightly distorted.  The images can be Centred or Offset by a specified shift.  Most Pict files have an origin at (0,0) and will, unless shifted or centred, be placed in the top left corner of the screen.

A Single file can be read; with Multiple files input the program will keep coming back to the Read image or Read binary dialogue (of particular use if the Analyse immediately has been checked).

Numerical sequence will only ask for the first file name then increment its numerical suffix (any complete sequence of numbers should work, can start from any value or just the name - Bin, Bin1, Bin2 … Bin9, Bin10 etc. or Bin.008, Bin.009, Bin.010, etc.).

Multiple files and Numerical Sequence are mainly of use in analysing binary images, but they can be used to preview a set of images using Read image.

Invert image is used where the features are white against a black background.  Objects are identified as black objects against a white background.  Inverting a grey tone or colour image is unlikely to produce sensible result.

.i.Read options (Image menu)

.i.Analyse (Image menu)

Analyse	Analyses an image a binary image according to the options selected in Anal opts.  It can be used with grey tone or even colour images, but this may give unexpected results.

The features on the displayed binary image are located and measured.  These can be saved as a data file and it can include the co-ordinates of the boundaries.  A single line of pixels is cleared round the edge of the screen in order to find objects touching the edge.  The screen is scanned line by line from top to bottom to find the first black pixel that indicates the start of an object.  The boundary of the object is then traced looking for the next pixel in either the orthogonal or diagonal direction.  The rest of the analysis is skipped if the mouse button is pressed.  The resultant boundaries are displayed as they are found.

The data limits from Data Format (Data menu) are used if co-ordinates are being saved.  If not, then the data limits will try to expand dynamically, providing there is free memory.

.i.Analyse options (Image menu)

Anal. opts	If .i.Save co-ordinates; is checked, a data file of co-ordinates will be opened immediately you start to analyse;  you will also be reminded to specify a Magnification (or Calibration) and section .i.Thickness;.  If Analyse immediately is checked then as soon as a binary file has been read using Binary read it will be analysed.  Show ‘.i.End frame;’ dialogue is checked then after each analysis the normal End frame dialogue (Options menu) is displayed.  Find .i.nested features; will cause the image of each object to be inverted so that internal white features are now black and will now be located.  Any level of nesting is possible.  To do this the profiles of the objects are erased as they are found and the full display shown after the frame has been analysed.

Objects outside a specified size range can be rejected.  The full screen is 640 x 480 = 307,200 pixels, but there is a size limit of an object of 4096 points that is the about the size of a square 1000 x 1000.  The areas are also expressed as real values using your specified Magnification and Units.

Identifiers can be in sequence as the objects are found, or the Object ID (as in Options menu) can be called for each object, all the objects can have the same identifying number, or the IDs can be found from the overlap of objects with those in the previous section.

The co-ordinates can be Saved as found or after applying binomial .i.Smoothing; else the data can be halved by saving Averaging pairs of points.  The main effect of smoothing is on the value of the .i.perimeter; that can be considerably increased by the jagged edge of pixels.  The perimeter is therefore recalculated after smoothing.  A second stage of smoothing is possible when displaying a 3-D reconstruction from the 3-D Goodies menu.

A .i.single pixel; width line is turned into an .i.open; line (having been located as boundary with each point originally appearing twice in the data set).  A perfect line has zero area, but even an odd extra pixel gives the line a small area.  The tolerance of the line conversion can be set using the ratio of the square root of the area to the length.  If this value is too large or with odd shapes (e.g. ) the program may not be able to find the end of the line in which case the user can either save the complete boundary as an open line or keep the object as a closed area.

Chunky pixels indicates whether the pixels are regarded as infinitesimally small points or blobs.  For all but the smallest blobs this correction is small.  The calculations are done as points then corrected for pixel size (Corrected area = Area + Perimeter ÷ 2 +1, Corrected perimeter = Perimeter +3).  Unless made chunky, the area of a straight line of pixels will be zero although any bend will generate a small area.  Any recognition of linear structures is done after this correction.  With chunky pixels all objects have an area so it will be necessary to specify an appropriate ratio.  Frame IDs in sequence makes them match the frame number.

.i.Point mode (Image menu)

Point mode	This selects a range of object sizes (by area) that are to be treated as a set of points.  The default object ID for points is 251, since objects are automatically numbered use the range 1–250, but any value (1-255) can be selected.  The Maximum size is automatically made the Minimum size for Anal opts if none has been selected there.  The Diameter of a point is an average value that will be used when plotting on screen.

The range of sizes selected is from the minimum up to but not including the maximum.  Although the limits are specified in pixels, the areas, which are calculated from the co-ordinates, may have fractions of pixels, e.g. 3 non-chunky pixels in a triangle has an area of 0.5 pixels.  Single pixels are always ignored in the general analysis, but can be included in point mode.

.i.Calibrate (Image menu)

Calibrate	Images, individual frames or 3-D reconstructions can be calibrated.   It is also designed to produce separate x- and y-calibrations for images which may have non-square pixels.  These separate calibrations will be used to Measure images, but their average (geometric mean) is used every else in the program.  When capturing images, either the video camera or frame grabber may distort images.  In general you should correct the images (e.g. using Scale and Rotate in the NIH Image program) before segmentation, rather than correcting data later.

An alternative calibration routine can be obtained if the Command key is pressed when Calibrate is selected or the digitising tablet is being calibrated.  This also enables the calibration to be taken from the screen.  Whilst the sequence of images is being captured you should take an extra file of a standard (e.g. stage micrometer or a ruler).  Enter the length of the line using your specified units then use the .i.mouse; to specify this reference length on the image.  Move the cursor to the start, press and hold the mouse button until the line stretches to the end.  Since many images do not have the same magnification in the X-  and Y-directions the calibration should be made in 2 directions and the mean value used if the two values are quite close.  The value obtained is also shown in Magnification (Options menu).

Calibrate reads a Pict image (binary, grey tone or colour) if none is already displayed and lets the user calibrate the system using mouse or tablet.  The same offset and scaling will be applied to this calibration image as the other binary images.  It is normally inappropriate to use the Scale to fit option when doing measurements.  Enter the calibration length (this uses the units you selected from the Units menu), click OK then draw your length on the image by pressing the mouse at one end of the reference and holding the mouse down until you get to the other end.  When you release the mouse the value is displayed.  Alternatively it possible to just click each end of the line, providing the first point does not move to create a short line.  It then takes repeated measurements (click More) and averages the resultant magnification or scale (click Done when complete).  You can also type a value in the dialogue, but if you just want to enter a value it would normally be easier to use the Magnification/Scale dialogue in the Options menu.

This procedure can also be used to calibrate an ADB digitising tablet.  If the tablet is active (IO config. in File menu) the calibration will be appropriate to the tablet and not the screen.  It is also possible to enter a calculated value for the Magnification/Scale (Options menu).

.i.Movie (Image menu)

Movie	This reads a sequence of Pict files.  If the files have a numerical suffix, a complete sequence can be read in automatically.  Read a single image (Read image) then make a Window (File menu) and reduce its size to match the image, then read the complete sequence.  The options (Scale, Invert, etc.) are the same as described above.  Each image has to be stored in memory, so use a small window and increase the memory alocation for MacStereology (see section 2.4).

.c.6.4	Tablet menu

These options refer to input from a .i.digitising tablet;, but Tablet scale can also be used with any data file to change the size and position of the display.

When the tablet is active (IO config in File menu), the puck or pen on a standard ADB tablet will change from tablet mode to cursor mode when the cross hairs cursor on screen moves up into the menu bar (the position of the cursor will jump to wherever the absolute position of it on the tablet would indicate) and revert back when passing out of the menu bar.  Providing the boundary of an object starts within the screen area, objects can be drawn which overlap the menus, as long as the Read button is kept pressed.

If an image from a Pict file is displayed, you can use a digitising tablet to trace boundaries on this image, but the graphics will use patterns not grey/colour tones.

.i.Step size (Tablet menu)

Step size or

Point diam	This is the distance (in mm) between data points on the tablet.  This means that points from the tablet are approximately equally spaced.  It is also used to determine the range for detecting a closed object.  (The keyboard equivalent J can be thought of as jump).  Step size can be specified individually for each identifier using Object ID (Options menu).

If using Point mode, then this item changes to Point diameter and indicates the display size of points at your Magnification and using the selected Units when collected in Point mode.

.i.Tablet wait (Tablet menu)

Tablet scale	This enables the graphics to be scaled and positioned appropriately on the screen.  The large square represents the tablet and the shaded rectangle the part that will be displayed on the Macintosh screen.  Use the .i.mouse; to drag the shaded rectangle to the required position.  Any data off the edge of the display is not lost and it will not upset any calculations or subsequent displays.

The X- and Y-offset enable values to be displayed relative to a point other than the bottom left-hand corner of the tablet.  The values here use the Units and Magnification you have specified.

There is no need to call Tablet scale to move the display since the position of the graphics can be .i.dragged ;using the mouse (may need to press command key ⌘) without calling Tablet scale.  However, be careful not to move the display if you are using a printout of the co-ordinates for serial section alignment.

Tablet scale can be used to change the size or position of the displays for dialogues Edit data., Align, Linkage, Show page and Pict File.

.i.Tablet scale (Tablet menu)

Tablet wait	This sets the delay between end of drawing one object and the start of the next.  If this time is too short you may find that the computer will think that you are starting the next object before you have had time to take your finger off the button at the end of the last object (default 0.25 sec).  The beep when the tablet becomes active again may be suppressed by turning off Beep points.

.i.Input reset (Tablet menu)

Input reset	Re-initialises the digitising tablet, and the other variables set from the Options menu associated with data collection, back to their default values.

.i.Show X-Y (Tablet menu)

Show X-Y	The values of X-Y co-ordinates can be displayed, near the top left corner of the screen, whilst data is being collected from the digitising tablet.  The data is scaled by the Magnification and uses your .i.Units; and .i.Precision;.

The action obtained depends whether you are using a digitising tablet connected to a serial port or via the Apple Desktop bus (ADB):–

a.	With a .i.serial-line tablet;, Show X-Y displays values only during data collection.  At the end of an object the values change to show the centre of gravity of a closed object (or mean x and y of an .i.open; object).  If you start a measurement by pressing any of the cursor buttons other than the Read button, then only the x,y co-ordinates will be displayed (until another button is pressed) and this data will NOT be added to your data file.

b.	With the .i.ADB tablet; the values are continuously displayed.

.i.IDs in order (Tablet menu)

IDs in order	This is similar to the option in Analyse opts (Image menu) in which the object ID is automatically incremented for each object.  During data collection the ID is set equal to the next object number immediately after an object is completed; it can then be changed before measuring an object using Object ID.

.i.Point mode (Tablet menu)

Point mode	Data from a tablet can be input one point at a time and a blob displayed for each point*.  Use the Cursor/Pen to pick off all points of one type.  When in Point mode and the tablet is active, there is a warning in the menu bar.  A set of blobs counts as a single object and will be displayed with a circle of the same size on screen.  You can have several IDs for different blob sizes or colours (e.g. nuclei, mitochondria, vesicles, etc.).  Record a series of similar points by clicking the Read button on each point, then complete the group of blobs by either a Close or Open command (both have the same effect), using buttons or “C” or “O” keys or Stop in the Point mode menu.  Cancel (by button or “X” key) deletes one point at a time and not the whole sequence of points.

The diameter of the blobs displayed can be specified from the Object ID or Point diam (this item is Step size if point mode is not selected).  During data collection the same variable is used for step size and point size so changing one changes the other (but in the data file each object has its own size).

The point diameter is saved as part of the data for each object (group of points), so need not be the same for all objects of the same ID.  The point diameter can subsequently be changed using Edit data (Data menu), and any profile can be switched between Closed, Open and Point mode (this recalculates the derived values).  The Area and Perimeter for points are the totals for all blobs in that set.

In 3-D reconstructions the points are scaled to size and can retain their circular shape (i.e. assumes that points are spherical) or shown as tilted disks.  If Show point surface is chosen (Select dialogue, see below) then the blobs are shaded as though they were spheres.  Note that all blobs within one object will have the same size, colour and shading.

.i.Beep points (Tablet menu)

Beep points	This has three actions.  It enables or disables the beep when each value is collected in Point mode and  in Measurements.  It also stops the additional beep after drawing an object that would indicate that the tablet is ready for the next object.

Offset	Enables features within a frame to be moved.  Thus if the parts of structure are too large to fit on the tablet the micrograph can be moved and offset by a mesured distance, or by a real distance using the Magnification.  This is useful where fine detail of a large object is required, e.g. in neuroanatomy.  Using this Offset modifies the stored data, whereas Offset in the Edit Data dialogue (Data menu) adds array of offsets, but does not modify the data.

.c.6.5	Options menu

These items are mainly used during data collection, either from Pict files or using a digitising tablet.

.i.Obj. ident (Options menu)

Object ID	The profile of each .i.object; is identified with a number (1–255) and you can also include an .i.object name; to identify what feature was being drawn.  (Objects are also numbered 1, 2, 3… within each frame, but the identifiers can be in any order.)

During data collection there is no need to use the menu or press I to change the Object ID; just type the new value (the normal dialogue will be displayed) then press Return or click OK (e.g. ‘2’, ‘5’, ‘Return’ for ID=25).  You can also increase or decrease the ID using the ‘<’ and ‘>’ keys (or ‘,’ and ‘.’), or increase or decrease the ID as well as showing the dialogue using the ‘+’ (or ‘=’) and ‘-’ keys.  Details of whether it is point mode and the step size or diameter will be recalled when an ID is changed.

When using a digitising tablet the Step size can be adjusted to specify the spacing of the points around the object (distance in mm on the tablet).  The .i.identifier ;should be specified before the object is drawn.  If in Point mode this dialogue changes to Diameter that indicates the size of each point (see Section 4.2).

.i.Frame ident (Options menu)

Frame ID	Each frame can be labelled with number (1–255) and you can also include its name.  A frame consists of a series of objects and can have an additional reference number for later statistics or a short text message for identification.  The Frame ID (also .i.Magnification; and Thickness) can be specified any time before the frame is Ended.

.i.Magnification (Options menu)

Magnification This is the magnification or scale of specimen on tablet.  If the value is less than 1.0 then it will switch between Mag. and Scale (type a 0 or decimal point as the first character of the value to change it).  Hence it possible to work with micrographs (e.g. Mag. x 50,000) or maps (e.g. Scale 1:50,000).  See also Calibration (Image menu).

.i.Thickness (Options menu)

Thickness	This is used to indicate section thickness or spacing (in the specified Units) for subsequent 3-D reconstruction, volume calculations or stereology.  Each section can have its own thickness, but for 3-D reconstruction this can be changed globally using Position (3-D menu).

.i.End frame (Options menu)

End frame	Marks completion of data on current frame.  You can change the Frame ID and name, Thickness and Magnification.  Graphics for this frame can be .i.Print;ed or .i.Plot;ted as an aid for serial section reconstruction.  Any scaling will be ignored and the output will be the same size as on the tablet.  (Use Draw frame if other scales are required – a scale of ÷7 will make the output the same size as on the tablet).  Pictures on an .i.ImageWriter; are scaled to give correct aspect ratio without clicking .i.Tall adjusted; in .i.Page setup;.  If the exact size is not obtained

on your .i.LaserWriter ;it is possible to change the X- and Y-scale factors that are stored in the resource fork as STR# 400, but you will need to use a .i.Resource Editor; (.i.ResEdit;) to change them.

If data is being Saved into a file, the complete file will be written after each frame is Ended.  Hence, Quitting may not need to ask if you want to save the data, and if there is a crash, such as may occur from a power failure, you will lose little or no data.

.i.Cancel (Options menu)

Delete obj. or

Reopen frame	Delete the last object from the data file.  If there are no objects on current frame, Delete object lets you re-open the previous frame (no objects are cancelled).  Whilst you are still using the tablet to draw an object use Cancel button on the cursor or X.

.i.Cancel frame (Options menu)

Delete frame	Cancels all data on the current frame, or after a frame has been Ended the data on the previous frame.

.i.Clear screen (Options menu)

Clear screen	Erases the screen.  If a binary Pict file is being displayed, this will be retained.  Use Clear image (Image menu) to remove it.  If a Pict file is being shown this will be redisplayed.  The Clear key on the keyboard can also be used.

.i.Auto clear (Options menu)

Auto clear	Erase the screen before drawing each object or next 3-D display.  Use Auto clear if you are measuring a series of objects whose positions are irrelevant (e.g. areas of peaks from chart paper).  You should also select Auto clear for 3-D reconstructions since the screen is normally cleared before each display.

.i.Fill objects (Options menu)

Fill objects	Fill each closed object with pattern or colour that depends on its ID number.  Each object also has its number placed at its centre (N.B. for some irregular objects it is possible for the centre of gravity to be outside the boundary of the object).  The fill colour/pattern is determined using .i.Select (3-D menu);, but it defaults to reasonable a range of colours/patterns if you choose .i.Reset 3D (3-D menu);.

.i.Pen size (Options menu)

Pen size	Drawings from tablet can use a thicker pen (may be useful when photographing the screen).  It is also used in the 3-D plots.  A pen size of zero will leave only the shading (may be appropriate for a 3-D display).  Fractional pen sizes can be entered, but these will only be used when printing on a LaserWriter or in a .i.PostScript file;.  If the pen size is zero it will remove the outer line of objects, but when this would result in nothing being drawn, such as an unfilled profile, a pen size of 1 is used.

.i.Label objects (Options menu)

Label objects   This sub-menu can be used to label objects in the graphical display of a frame to use the object number (1, 2, 3… in each frame), or the object identifier number (which can be any value 1–255), or to have no label at all.  Objects with an ID=0 will not normally be displayed; use Edit data to change the ID.

.i.Draw frame (Options menu)

.i.Redraw (Options menu)

Draw frame	Draw, .i.Print; or .i.Plot; the profiles from any frame in the current data file.  Draw frame and Redraw are only possible if the co-ordinates for each object have been saved.  Providing the tablet it not active, the ‘<‘ (or’,’) and ‘>‘ (or ‘.’) keys display the previous or next frame.  ‘+’ (or ‘=‘) and ‘–’ keys move to adjacent frame, but also display the dialogue.   If a binary image is being displayed this will be drawn behind the frame.

Redraw	Redraw profiles for the current frame.  The current frame is the last one displayed or changed, e.g. the last digitised or edited.  This can be used to show a single frame after a 3-D reconstruction or may be necessary after using a desk accessory if you are recalculating the display (see Mac config in File menu).

Measure	Values from individual sections or 3-D reconstructions can be recorded using the mouse.  Values are stored and Save as can be used to make a text file, c.f. Text file (File menu).

When Measure is selected, if there are already measurements stored, you can delete all of these or add additional ones.

Moving the mouse shows the x-y values, scaled by the magnification, plotting scale and offsets and using the specified units.  Clicking the mouse button stores a value and marks the point with a cross.  Clicking then dragging marks a line whose length and orientation (in range -180 to 180° or 0 to 360°, where 0° is east, positive anti-clockwise) is also recorded.  A line followed by a second line defines an angle difference.

Measurements on sections record which frame is being displayed, whilst with a 3-D reconstruction the Rotation and Tilt is recorded.  Measurements on a 3-D reconstruction are in the plane of the screen and therefore do not follow features in 3-D.  Measurements will not be accurate if a perspective view is plotted.

Holding the Command key down and clicking offsets subsequent data relative to this point.  Delete a point with 'X' or Delete key.

Meas. opts.	Determines which values are displayed in Measure, but all values are stored and can be output to a file.

.c.

6.6	Units menu

Select the units required for data measurement.  The data is stored as absolute values and can be subsequently printed scaled for any units.  If the Magnification is x 50,000 you will probably want to use µm, whereas if the scale is 1:50,000 the units could be Mile or Acre.  A thou. (or mil) is 1/1000 of an inch.  If you change the units you normally do not need to change the .i.Precision (Data menu); of the numbers, except possibly when making a .i.TEXT file (File menu);.

If Acre or Hectare is chosen then areas will be shown using that unit and any linear measurements will be given using the square root of the unit (shown as √Acre or √Hectare)

If a unit not on this list is required, select the other… item, then define your unit in terms of the previously specified unit, e.g. to specify a Chain as your unit, select Yards, then Other…, then enter 1 chain = 22 yards.  Indicate if it is a Linear or Area measure.

.c.6.7	Data menu

.i.Data format (Data menu)

Data format	This enables the size of the .i.data file; on disk to be adjusted to match your requirements.  If only a limited amount of data is being collected then the size of the resultant file can be substantially reduced.  The window indicates the current limit and the range that is possible.  Total text indicates the number of characters in all the labels for frame and object identifiers and also .i.Frame order (3-D menu);.

The Data format option is unnecessary if data is not being saved to disk and can be done at any stage if co-ordinates of each object are not being saved.  However, if you are saving co-ordinates, this Data format dialogue will be automatically displayed (together with the section .i.Thickness;, and .i.Magnification;) before the first data is saved to disk.  This is to allocate the appropriate starting point on the disk for storing the co-ordinates in the data file.  Always specify a generous limit, since your specified limit cannot subsequently be increased (see Formatting data Section 5.1).

.i.Edit data (Data menu)

Edit data	This displays the data and enables changes to be to the stored values.  Changes are made permanent by clicking OK.  On the right hand side, the upper window shows all the data for the frame whilst the lower window has a choice of displays:– the object being manipulated, objects whose IDs are repeated (needed for 3-D surface reconstructions), the last frame or the next frame.

The graphics can also be dragged, zoomed and centred.  Select an object in the current frame by number or by clicking its number inside the graphics window.  The size of the display can be changed using Tablet scale (Tablet menu) even though you may not be using a digitising tablet.  If co-ordinates are not being saved, only the numbers will be plotted at their centres of gravity.

Magnification ;and section .i.Thickness of a frame ;can be changed.  The other parameters can be edited for each object in that frame.  .i.Closed object;, Open object or Points flags the object as that form and recalculates the derived parameters (this may change the value for Perimeter and Area).  Point diameters are saved as integers so the displayed values may differ slightly from the true value.  In point mode the Area and Perimeter are the sum for all points at the specified diameter.

If saving co-ordinates, it is possible to Get IDs from the objects on either the last or next frame.  The search is made between the centre of gravities of all objects on one frame and the boundaries on the other then repeated in the reverse direction.  Any objects not found IDs are given a new ID.  For a Surface reconstruction all objects must have a different ID.  Use Show rpt IDs then click item again to resolve duplicates.  The IDs of objects in Point mode are not changed.

Use .i.Frame ID (Options menu); or .i.Object ID (Options menu); to change the names of the identifiers, and .i.Precision ;to change number of figures in data.

Offset moves individual features within a frame (see Offset in Tablet menu).  Since groups of features are normally moved the offset applies to the specified object number and subsequent ones in that frame.

.i.Precision (Data menu)

Precision	Precision indicates the number of significant figures when the numbers are printed or displayed on screen.  The menu shows the selected number of precision (default 4 figures, e.g. 123,400 1234 12.34 or 0.01234).  N.B. The displayed precision in no way affects the actual data that is always stored at the full precision of that data format (about 7 figures for a decimal number, but most values are saved as exact integers in tablet co-ordinates of 50 µm).

.i.List data (Data menu)

List data	Similar to print data but lists to screen of the Macintosh.  Click OK button to remove listing (or anywhere if whole data fits within the screen) or Save as a file as in Text file (File menu).  If using a Macintosh with a small screen and displaying data to high Precision, you may need to use a smaller Font.

.i.Volumes (Data menu)

Volumes	This displays the Volume for each object identifier using the Cavalieri’s method of Volume = ∑Area.Thickness.

Length is the distance between the centre of gravity of objects on adjacent sections.

The Surface area is measured from the plotted surface and hence depends on the plotting resolution and any data smoothing.  Since this involves a surface reconstruction it is relatively slow and not enabled unless checked.

The data can be Saved as a file as Text file (File menu).

.i.Font (Data menu)

Font	This sub-menu will show the fonts installed in your system.  Installed sizes will be shown with their values outlined in the Font size menu.  Some fonts are bit patterns so will not be printed at full resolution on a LaserWriter.  The font name and size are saved in the data file, and it will be used if it is available, otherwise the previously selected font will be used with Geneva as the default.  If the font or size is too large, particularly if the data displayed to many significant figures, the text may not fit the available space - try reducing the Font size or turning the page sideways.

.i.Font size (Data menu)

Font size	Shows a range of possible font sizes with those available on your system outlined.  The small sizes 6, 7 and 9 may not be available, but produce adequate results if the 12, 14 and 18 point sizes are installed.  In dialogues that show a reduced sized display the next smaller font size is used.  (The items in this menu can be changed using .i.ResEdit;).

.i.Colour table (Data menu)

Colour table	For a colour Macintosh this will display all the colours in use, a sample spectrum (if the last plot was a 3D reconstruction) and the scale of patterns which available for dithering colours.  Dither is used where a colour that is not available is approximated by mixing the 2 nearest colours as a pattern of dots of the two colours.  64 Patterns are available for dithering, but you can restrict this to the simpler patterns or even just the full tones.  For a 256-colour Macintosh there are probably sufficient colours available without using any dither, and any dither would be almost invisible.  Use of 24- or 32-bit colour is not normally necessary and may slow down the display.  When analysing Pict files a colour scale for 250 IDs is set up.

The wrong colour lookup tables are occasionally applied to the screen, e.g. colours for a 3-D reconstruction is applied to the display of the profiles in a frame (typically all objects are coloured red).  Calling Colour table then clicking OK recalculates the colour tables.  Input from Pict files always uses the System palette, whereas MacStereology uses indexed colour for reconstructions in order to give the greatest range of colour tones.  Pict files are saved with these colour tables and these can now be read by many applications.

.i.Start movie (Data menu)

Start movie or

Add to movie	Starts a new sequence.  The image on screen is saved using the size of the window or full screen currently displayed for this and all subsequent images in the sequence.  This menu item changes to Add to movie after the first image has been saved.

.i.Data format (Data menu)

Delete image	Removes the last image from the sequence.

.i.Run movie (Data menu)

Run movie	Displays a Sequence of images or one made using Movie.  The speed of display is controlled by the keys 0–9.  ‘0’ stops the movement, the other keys slow the display (such that the time between frames is the value squared in ticks, i.e. 60ths of a second, e.g. ‘5’ is 25/60 sec per frame).  The ‘+’ (or ‘=’) key increases and the ‘–’ key decreases the time between images by one tick.  Oscillate or letter ‘O’ starts or stops oscillation, ‘F’ and ‘B’ shows sequence in forward or backwards direction, ‘.’ or pressing mouse button quits the display.  A sequence can be viewed at any stage whist you are adding frames; further frames are added to the end of the sequence.

.i.Erase movie (Data menu)

Erase movie	Deletes the current movie and releases the memory it used.

.i.Sequence (Data menu)

Sequence	This is used to make an animated reconstruction.  Click the text of Frames in sequence to see maximum to fit available memory, then subtract a few for safety.  The limit for a Sequence of 3-D reconstructions is only approximate so may vary according to how much computation is involved in making the image (e.g. a Surface plot takes much more memory than a Layers or Wireframe reconstruction).  Sequences can be added to an existing movie.  The last frame need not be saved if using a cyclical range (e.g. 0–360 rather than 0–30).

The reconstruction can make a movie (stored in memory), made into separate Pict files or as a single Pics file.  Pics file can be used by various animation programs including ‘Pics Player’ in ‘AfterDark’ and NIH Image.

A sequence can be made showing the build up of the sections, but only if the Order (3-D menu) is either forward  (1-n) or reverse (n-1).  The required number of frames will be displayed (if possible).  Pressing the Command and Option keys changes this to ‘Build up’ / ’Build down’ / ‘Strip up’ / ’Strip down’.

.i.Oscillate (Data menu)

Oscillate	Enables the sequence to run forwards then backwards through the sequence rather than just forward.  For realistic animation, a small angle between images is desirable, so, for instance, it is possible to cover 0–30° in 2° steps and show the result as an oscillating display.

.i.Fade (Data menu)

Fade	Lets the images of a sequence merge into each other.  The number of stages can be varied, but fading images greatly slows the display.  Fade=2 is useful for sequences of images such as Muybridge's running horses in which there is a sequence of photos with quite a large step between frames.  A movie of only 2 or 3 images can be shown with Fade=16 to show the parts of a reconstruction.

.c.6.8	3-D menu

.i.Wireframe (3-D menu)

Wireframe	A simple line drawing of the 3-D reconstruction, suitable for use with a .i.digital .i.plotter;;, since all masking is precalculated such that only the required lines are sent to the plotter.  The implementation of the masking is very simple so may show dotted lines that should be solid.  The Pen size (Options menu or Lighting dialogue) controls the line width, but fractional line widths can only be printed on a LaserWriter.

.i.Layers (3-D menu)

Layers	The profiles of closed objects can be filled with a pattern.  Use this option for the quickest display on screen or for sending to a printer.  The objects on a layer are filled with the selected pattern and have an outline of either the Highlight or Shadow colour as chosen in Select (3-D menu).  A Pen size of zero will remove the boundary line.  Line colours are chosen to provide contrast with the Fill colour/ pattern (in this example changing from white to black as the pattern gets lighter).

.i.Surface (3-D menu)

.i.Half-tone; on .i.LaserWriter;       Bit Map print

Surface	A 3-D display calculates the surface of the 3-D structures and shades them according to their orientation to the light sources.  The size and shading of these facets can be varied and Pen size (and Filling) controls whether the layer lines are visible.

.i.Viewpoint (3-D menu)

Viewpoint	This specifies the direction for viewing the reconstruction by indicating the .i.Rotation ;about the vertical axis (through centre of gravity of first object in data file) and the .i.Tilt ;of this axis towards you.  Any value of Rotation or Tilt is possible.

For an isometric view (as though the structure is being viewed from a great distance) leave the Distance as zero.  For most reconstructions this isometric view is most appropriate.  If a full .i.perspective ;view is required, enter an appropriate Distance.  Expect strange results if you try to view an object from within.  With a perspective view it also possible to specify the Height of your viewpoint.

A .i.Stereo pair; option makes two separate reconstructions at the specified spacing and an angle of the Rotation ± 1/2 Angle.

Anaglyphs can be made by capturing separate red and green/blue or cyan images and adding them together.  A monitor with 24/32-bit colour is required to see the result.

.i.Lighting (3-D menu)

Lighting	All items in the Lighting dialogue refer to the whole reconstruction, whereas those in the Select dialogue refer to each ID.  Specify the direction of the light sources for the Surface 3-D reconstruction.  Change the limits to compress or expand the grey scale (0–63) and it is possible to highlight the extremes if the minimum is less than zero of maximum greater than 63.  You can use the mouse to drag the extremes, providing you start within the grey scale window.

The buttons for specifying the number of tones are normally not necessary when using full grey scale/colour tones display on a Macintosh II (See Colour table in Data menu).

The Depth shading % indicates the fraction of shading between only the orientation of the light (0%) to just distance from observer (100%).  Mark object edges may be useful to make the display clearer, particularly if patterns are being used (e.g. on a Macintosh Plus)

Translucency indicates how transparent/opaque objects are when these modes are chosen in Select.  This plotting mode can make parts of a reconstruction translucent so that internal features can be seen through.  Use Select to change the mode;  by default all objects are Opaque.  The degree of translucency can be set from the Lighting dialogue which sets a pattern for this masking (0 = opaque, 32 = half transparent, 63 = transparent).  On replotting the reconstruction the opaque features will be plotted first (always with a white background), then converted to a black and white mask, followed by a full plot and finally partial masking of the translucent items.  This means that translucent items may mask each other and setting an ID to non-opaque will only make it translucent relative to the opaque features, not other translucent features.  If you look closely the result with the default mask (translucency = 32) you will see that it copies alternate pixels.  Translucency is possible, though it does not look so well, with black & white or dither patterns, but is meaningless for a Wireframe reconstruction.  To use it you must have the normal screen refresh (Mac. config. not Recalc refresh) and it will only be displayed on screen and not to files, printing or plotting.  Use Copy image or Save image (or Output pixel Pict file) to transfer or print the reconstruction.

Interpolated shading indicates how each facet in a surface plot is subdivided. Facets in a Surface reconstruction can be subdivided to produce smooth variation in shading to avoid the Mach band effects (where eye sees neighbouring colours with enhanced edges).  It is best used with a full colour/grey tone reconstruction.  Select a value (1–9) from (e.g. 5 divides each facet into 25, whilst 1 does a standard plot).  For best use of this option make the facets approximately square by adjusting resolution (typically 3–6) and probably reduce the Smooth shading (Goodies dialogue).  To see a smooth surface with no layer lines set the Pen size to zero, and it may be necessary to remove Filling (Select dialogue) which uses the same boundaries.  Note, this procedure slows plotting considerably.  Holding the mouse button down will stop plotting, but only after the facets for the current layer are complete.  When using a LaserWriter you may need to adjust the Halftones dots/inch (File options dialogue) from the default (0=default for printer) to say 37.5, but the result will be rather large half-tones.

.i.Position (3-D menu)

Position	The .i.Local .i.magnification;; and .i.Absolute magnification; (absolute mag. = local mag. x specimen mag.) are displayed and either can be changed.  (These will be the same if the original magnification of the specimen was 1.0).  A local magnification of x1 will normally give a reasonable .i.plot; on screen, since the features will be the same size as on the tablet.  The output to .i.ImageWriter; and LaserWriter printers or digital plotter will be scaled to give the correct dimensions.

.i.Layer spacing; enables you to varying the spacing between sections to produce an exploded view.  Unlike .i.Thickness;, which can be specified separately for each section, this is a global value that will be applied to all sections.  A zero value for Thickness indicates that the specified thickness for each layer should be used.

.i.Resolution ;indicates how many of the original co-ordinates should be plotted.  For high resolution set to 1, but for more rapid plotting, at lower resolution, increase to about 5, i.e. plot every fifth point.  (For maximum resolution of the .i.Surface; plot choose a value of 0, so that each object will then have 127 facets on each layer).

X- and Y-shift move the origin of the plot.  The plot is centred on the centre of gravity of the complete reconstruction.  Dragging ;the plot about the screen also changes these values and is generally much easier to use.  .i.Rotation;, .i.Tilt;, X-shift and Y-shift all work relative to the centre of the reconstruction.  Where output will be to an external device, use .i.Show page(3-D menu); to see how it will fit.

.i.Select (3-D menu)

Select	All items in the Select dialogue refer to each ID, in contrast with those in Lighting dialogue which refer to the whole reconstruction.  Select the object identifiers to display and their pattern, grey tone or .i.colour.  The identifiers that have been used in current data file are listed.  Shading/patterns apply either to an object identifier (.i.Shade by ID;) or to all objects in a layer (.i.Shade layers;) between the limits shown in the Range boxes.  This range shows the shading/tone/colours for the bottom and top layers;  intermediate values will be used for the intervening layers.

To change the value of .i.any item (including Revert) for ALL IDs hold the .i.Command ;key (.i.⌘;) down whilst changing the value (e.g. ⌘ and click Fill will change the filling for all objects to the mode shown).  Similarly the Shift and Option keys will change for the current and either all previous or subsequent IDs.

Revert will restore all the values for that ID to what they were before Select was called.

The Plotting modes indicate how any features with a particular object identifier will be displayed:–

Don’t plot means that the objects will not be plotted nor obscure other objects.

Dots or Dashes enables the only the outline of an object to be indicated, but it does not mask any objects within.

Line+dots or Line+dashes displays the visible boundaries of the objects with the hidden boundaries shown in dots or dashes.  Objects in these modes may mask other objects.

.i.Solid; displays the visible boundaries of the objects, but hidden features are masked, other objects are masked.

Don’t mask indicates that the whole boundary is drawn, whether or not it is hidden.

Surface will calculate a surface joining the layers of a structure.  The surface will end in a conical cap or open surface as specified in the Linkage.  If the objects are also Filled it is possible to put a flat cap on the end of a structure.  Surface is used only with the Surface 3-D plot and will be treated as Solid with a Layers or Wireframe plot.

Since each object has its own plotting mode, mixed mode plotting is possible.  The effect of this will be to highlight or hide parts of a structure.

Fill indicates whether a closed boundary should be filled with the specified pattern.  This can be used to show the outline of a structure without masking internal detail.  Note that filling with white

(shade=63) is different to not filling.  A boundary can be changed from open to closed using .i.Edit data (Data menu);.

If dots or dashes are plotted these are calculated from the actual data co-ordinates, so will line up if you make a stereo pair.  The spacing of dots is determined by the original .i.Step size (Tablet menu); when the data was collected and the Resolution (in Position dialogue) for plotting.

.i.Shade ;indicates the grey tone (0–63).  For the .i.Macintosh II; full grey shading and colour tones are possible, otherwise dither patterns are shown.  .i.Colour ;can be used to select colours for a colour Macintosh II or a colour plotter.  .i.The Highlight colour; is the main colour (0=black, 1=red,  2=green, 3=yellow, 4=blue, 5=magenta, 6=cyan, 7=white).  The .i.Shadow colour; is used with patterns, but also indicates the full span of colours that will be used for surfaces or layers.  Alternatively, click (and slide) on the displayed grey/colour scale to select a shade.

With a black and white screen, juggling Highlight and Shadow can invert a pattern.  A simple pattern is obtained if the Shade is a multiple of 8 (e.g. 8, 16, 24, etc.).  The colour spectrum displayed on a colour Macintosh may show that some combinations of colours result in a dull grey in the mid tone region.

Features for an ID can be made Opaque or translucent.  If the resultant plot contains a mixture of modes the reconstruction will be plotted in two stages.

Spherical points is used with data in point mode to show whether points were spheres or thin discs (displayed as ellipses according to Tilt, but not adjusted for perspective views).

Fixed colours indicates that all objects of an ID will be shown with the same colouring/shading, but if that object is points the text will change to Show point surface that indicates that each point will be plotted with surface shading to give it a more solid appearance. points will be plotted as shaded spheres/ellipses.  Colour by Layers uses the specified range to make each layer of the reconstruction a different shade/pattern between the specified limits.  The boundary of each object is made using either the Highlight or Shadow colour/shade, whichever gives the greatest contrast.  This means that the boundary will probably change about half way through a plot when the calculated shade passes 32.  If the Range does not include 32 then there will be no change.  With Fixed colours the boundary will stay the same for all layers.

Reset restores the default values for that ID (or range of IDs).  The Highlight and Shadow will be colours if working in colour.

.i.Order (3-D menu)

.i.Linkage (3-D menu)

Order	Enables you to change the order in which .i.frames; are displayed.  Apart from showing the full sequence in forward or reverse order, it possible to show any frame in any order, including repeats and omissions, e.g. 1 3-6 8*3 12-9, but be careful of the effects of this if you have specified a Linkage.

Linkage	Enables you to change the way objects on one layer are linked to those on another layer for the .i.Surface; 3-D display.  For simple structures it is not necessary to specify any change from the default linkage.  For correct display of surfaces it is essential that each object in a frame has a separate ID (You will be warned is you try to display such a file – use Edit data to see repeats).

As you move the cursor around the 3 windows the numbers will be highlighted as you pass over the centre of an object.  To select objects for linking, click them in the central window.  Select or deselect what it links with by clicking the objects in the other 2 windows.  Linked objects are shown shaded.  Where a link is broken an ID of 255 may be shown.

Each object has associated with it two pointers to indicate the link with the identifier number of an object on the previous layer and on the next layer.  By default it is assumed that objects link with those of the same identifier on both the previous and the next layers, so you only need to change this when an object splits, joins or is cut.  To establish the surfaces between two layers the links are checked both forwards from the first of the layers and backwards from the next layer.  Examples:–

a. If object ID=1 on one layer splits to become two objects ID=1 and 2 on the next layer.  You need only click the object ID=2 links on the previous layer (the program will then automatically find that the object on the first layer has divided into two objects).

b. Objects ID=5 and 7 on one layer combine on the next layer to an object ID=9.  Select in turn ID=5 and 7 and for each click the object ID=9 on the next layer.

c. For an object that is cut off within the specimen leave the link to the next or previous layer as the same ID;  but for an object which ends click Cap (sets link to ID=0) in the next or previous layer and it will be drawn with a cone from a point (half the section thickness and the centre of the adjacent profile).  Caps are marked with a cross in the adjacent display.

Specifying a split or join displays the two surfaces merged by drawing the several overlapping surfaces between the single profile and each of the separate profiles.  This may look ragged unless using highest .i.Resolution (Position option);.  You can, however, define the joins exactly by drawing the parts of the single profile, corresponding to the branches, on top of the original single profile (see section on Linking objects section 5.7)

The 4 buttons in the top left corner relate to the whole structure.  You will be warned if you click any of them.  The group of 4 buttons in the lower left and right corners control all the links to the Last of next layer.

.i.Align (3-D menu)

.i.Output (3-D menu)

Align	The sections can be moved to bring them into alignment.  Adjustment is both for X- and Y-shifts and Rotation.  Adjustment can be made by typing the values or dragging the windows.  If the values are typed it may be necessary to click Redraw to see the effect.  The shifts indicated are in tablet co-ordinates, i.e. 50 µm steps.  If frames are rotated an extra block of data will be added to the end of the data file when you save the changes.  Any alignment changes appear in all displays, except Align and Edit, but the original values of the data are unchanged and will be used if you Print or List the data.

You can X,Y Shift and/or a Rotate the images.  Rotations done by dragging the images are about the centre of the display, hence will change the values of both rotation and shifts.

Dragging or rotating each of the 4 windows produces a different effect:–

To see any part of the complete display, you can drag the window showing the Composite view and centering, zooming in or out by pressing the Command, Shift or Option key then clicking the display;  none of these actions change the alignment.

Drag the window showing This section to move this one section relative to the other sections.  The actual movement will be shown in the X- and Y-shift and rotation boxes.  All sections will force the changes to This section to be applied to all the sections.  This can be used to Clip reconstructions at various orientations.

Dragging the Last section window will move the last section and all previous sections, whereas dragging the Next section window will move the next section and all subsequent sections.

Reset returns all the offsets and rotations back to zero.  A warning message is displayed first.

Clip	Cut-away views of the structure can be plotted by clipping any or all of the components.  The display can include all IDs and all layers.  Each ID can be separately clipped by dragging the lines to expose the required parts of the view.  Propagate values for several IDs by pressing the Command, Option or Shift keys whilst dragging limits or clicking a box or the value.

A default clip to the outer edge is setup if there are no values in the data file, and this can be re-established using Clip to limits.  The 3-D display can cut off completely at the limit or Plot full boundary with only the content clipped.

Clipping is performed for surface and output to a pen plotter by modifying the boundaries to the specified clip and trimming the ends of open objects.  This results in the cut surfaces being plotted as a plane in Surface mode.  To avoid shading creeping round the corners keep the Resolution (Position dialogue) to 1 and stop Smoothing of shading (Goodies dialogue).  For all other plots the full data is plotted with the clipping being done on screen using a mask.  Clipping slows down plotting to about half normal plotting speed.

Don’t clip turns off clipping without losing the values.

Data that has been realigned by shifting or rotating sections can be clipped, but this is not fully implemented yet and may not be in quite the correct position (limits are for the original data).  Rotating all sections by the same amount (Align dialogue with All sections checked) can be used to clip the structure at other angles.

The details of the clipping are appended to the end of the data file.

Output	This indicates where the 3-D reconstruction should be sent.  By default, it is made on Macintosh Screen (and it will default back to this after a plot has been made to another device).  Select the output device, then if you want to see how it will fit the page, exit with .i.Show page; button (same as choosing Show page from menu after OK).  Output to the .i.Printer;, .i.Plotter;, .i.Pict file; or .i.PostScript file; will be indicated by an icon in the bottom right of the screen.  Pict files may be either pixels (as is done in Save image and is like a MacPaint image) or vectors (‘MacDraw’ type image).  Pict files or Postscript files can use either the screen size or the paper size as defined in Page setup.  You can change the other 3-D options before making the 3-D .i.plot; on the printer/plotter, then print/plot using Wireframe, Layers or Surface.  Output to a .i.PostScript file; ignores all colour information.

The Plot sideways option is similar to the page orientation option in Page setup, but the plotting areas are not quite the same.  Printing of the three types of reconstruction can also be done directly from the Print option (File menu).

PostScript printing or files can use colour.  For a B&W printing the shades of grey are used against a white bacground.  A Pict file – vectors is best for use with a slidewriter since the full resolution of the graphics is stored.

The .i.HP 7475A plotter; has 6 pens that are assumed to be the colours 0 (black) – 5 (magenta) with 6 treated as blue instead of cyan and 7 as black not white.  You can, of course, put in any colour pen in any pen slot you wish.

.i.Reset 3D (3-D menu)

Goodies	This enables you to annotate your 3-D .i.plot;, including experiment title (from Data format in Data menu), file name, scale bar and rose to show orientation.  Each object in the reconstruction can be Numbered as given by Label objects (Options menu).

You can also select the .i.Smoothing ;options for the boundaries of profiles of an object when it is plotted as a 3-D reconstruction in any of the three modes.

Smoothing profiles can be used to reduce the detail on the data but do not smooth is there are very few points in the profiles (e.g. 4 points to give a square).

Smoothing shading applies only to the facets for a Surface reconstruction.  It can improve the appearance if a high resolution reconstruction is made (Resolution is 0 or 1 in Position dialogue), but is best left off (zero) when interpolating facets (Lighting dialogue).

Smoothing is by the binary coefficients (e.g. s=2 will substitute xi by (xi–1 + 2 xi + xi+1)÷4). For LaserWriter output of Surfaces a value for Smooth shading of 4 is useful to make the graduation of .i.grey tones; smoother.

Numbers objects adds annotation of object number or ID number to each layer of a 3-D reconstruction.  Its main use is in tracking down particular features when adjusting data.

Reset 3D	Change 3-D display back to its default size (local mag=1), position, viewpoint (rotation=30°, tilt=15°), lighting and shading.  The 3-D reconstruction will probably fit the screen.  If .i.Colour ;(.i.Mac config.; in File menu) has been selected then the default colours for objects will be included, otherwise only black and white are used. (See Defaults Section 7.2).

.i.Show page (3-D menu)

Show page	This is a print preview and will display a reduced size plot (always as Layers) of the complete page, file or screen, where the page size is the one selected using .i.Output ;(and if appropriate .i.Page setup;).  The image can be dragged or varied in magnification (same as Total mag. in Position dialogue in 3-D menu)  This will enable you to see the effects of changing the size and orientation of the reconstruction before hard copy is printed or plotted.

OK accepts any changes to position and magnification, but does not output the reconstruction (useful if you want to make another change, such as rotation, before outputting the reconstruction).  If you have selected a device other than the Macintosh monitor (Output in 3–D menu) there will be an icon in the lower right corner of the screen indicating that the next plot will be sent to that device.  You can then directly output the three modes of 3-D reconstruction.  Cancel forgets all changes made in the Show page dialogue and returns output to the Macintosh monitor unchanged.

.i.Black B-G (3-D menu)

.i.White B-G (3-D menu)

Black B-G or

White B-G	Change the background to black (or white).  To ensure that the display is visible some of the black or white colours may be reversed.  On a B/W (1-bit) monitor patterns and lines can also be inverted by reversing the .i.Highlight; and .i.Shadow; colours in the Select window.

.i.Goodies (3-D menu)

.c.Section 7 – Technical details

This section covers some more technical aspects of the operation and construction of MacStereology.  It may be of use if you run into difficulties in doing a particular image analysis, or wish to probe more deeply into how MacStereology is operating.

.c.7.1	Items which appear in several places

Magnification

During data collection:– In  Options menu and End frame (Options menu) and Data format (Data menu).  Magnification will be called automatically after the first object if Save coordinates is selected.

With a data file:– Edit data (Data menu) changes magnification separately for each frame.

Position (3-D menu) changes the size of a 3-D reconstruction that can be expressed as the Total magnification relative to the original specimen or the Local plot magnification relative to the size of the images/prints.  Total Mag. = Magnification x Local mag.  Total Mag. can also be changed from Show page (3-D menu).

Thickness

During data collection:– in Thickness can be changed from Options menu and End frame (Options menu) and Data format (Data menu).  It will be called automatically after the first object if Save coordinates selected.

With a data file:-  Edit data (Data menu) changes Thickness separately for each frame.

For a 3-D display the Thickness of individual sections can be overridden from Position dialogue (3-D menu); set to zero to use values from data file.

Step size

This is used only during data collection and Step size appears in Tablet menu, but when in Point mode (Tablet menu) this message changes to Point diam.  Step size also occurs in Frame ID (Tablet menu) and Object ID (Tablet menu) where the step size is for that ID only.

Point mode

Point mode can be called during data collection from Tablet menu and Frame ID (Options menu).  If called from Object ID (Options menu) it will be recalled each time that ID is used.

When analysing Pict files an equivalent command is used if a range is selected in Point mode (Image menu).  The maximum size specified for a point will automatically be the minimum size for the main Anal. opts (Image menu) if no size range is specified.

With a data file:– Edit data (Data menu) can be used to change any object to a set of points.

The Diameter of a point in Point mode dialogue (Image menu) is the same as in Frame ID, Object ID and Point diam (Tablet menu), but this item is only displayed if in point mode (otherwise Step size).

End frame

End frame is called from Options menu, but may be called automatically during analysis of binary images if selected in Anal. opts (Image menu).

Frame IDs in sequence

Frame IDs in sequence in Anal opts (Image menu) is the same as in End frame (Options menu).

Show page

Show page (3-D menu) is the same as Show page in Output (3-D menu) and shows the page according to the selected Output (3-D menu).  By default for Macintosh screen if no other device selected.

Print frame

Print frame (File menu) is similar to print in Draw frame (Options menu), but Print from End frame (Options menu) always prints at the same size as the original object as drawn on a tablet or from a binary file.

Plot frame

Plot frame occurs in Draw frame (Options menu), but Plot from End frame (Options menu) always plots at the same size as the original object as drawn on a tablet or from a binary file.

Print 3D

Print 3D Wireframe, Layers or Surface (File menu) is the same as selecting Output (3-D menu) to Printer (or LaserWriter, ImageWriter or whatever name your printer uses) then printing by selecting Wireframe, Layers or Surface (3-D menu) or from Show page.

Read opts

Read opts (Image menu) shows the options for Read image (for a grey tone or colour image), and Read binary.  These options also appear when calling Movie (Image menu).

IDs in order

IDs in order (Tablet menu) is the same as in Anal opts (Image menu).

IDs from last frame

IDs from last frame in Anal opts (Image menu) is the same as in End frame (Options menu).  An equivalent operation on data files can be called from Edit data (Data menu).

Save co-ordinates

Save co-ordinates in IO Config (File menu) is the same as in Anal Opts (Image menu).

Pen size and Auto clear

Pen size and Auto clear (Options menu) are repeated in Lighting (3-D menu).

Precision

Precision (Data menu) is the same as in Text file (File menu) and Measurements and Meas. Opts. (Options menu).

Sequence

In Sequence (Data menu) Rotation and Tilt take their default values from Viewpoint (3-D menu).  Distance, X- and Y-Shift take their defaults from values shown in Position (3-D menu).  Lighting takes its default from Lighting (3-D menu).

X- and Y-shifts and Rotation

Dragging the display of a frame changes the X- and Y- shift in Tablet scale (Tablet menu), dragging a 3-D reconstruction changes the offsets in Position (3-D menu) and dragging an image from a Pict file changes the offsets in Read opts (Image menu).

Separate X- and Y-Shift and Rotation are used in Align (3-D menu) and these refer to individually frames.

Read opts (Image menu) has a separate X- and Y-Shift for images from a Pict file.

Output

The file of a 3-D reconstruction as a Pict file of pixels from Output (3-D menu) is the same as displaying the reconstruction then calling Save image (Image menu).

Clear image and Clear screen

Clear image (Image menu) only clears an image from a Pict file whereas Clear screen (Options menu) clears the display and not the image from a Pict file.  if Auto clear (Options menu or Lighting in 3-D menu) is set Clear screen is called before each object is started during data collection or before each 3-D display.  If not set then during data collection the display is cleared before each frame and 3-D displays redrawn on top of any existing display.

.c.7.2	Data structure

The size of the data file depends on the what is being measured and how many values have been collected.  The limits are specified using Data format (Data menu).  If the co-ordinates of each object are being saved the data file can be quite large.

0.5 kbyte	All the general variables, flags and pointers.

Up to 4 kbyte	Summary of the frames, 16 bytes per frame up to 255 frames.

Variable	Data from each object, 32 bytes per object up to 32767, but number may be limited by free memory.

1 kbyte	Pointers for object names and frame identifiers, up to 255 IDs.

~1 kbyte	Colours and grey tones for 3-D plot.

Up to 8 kbyte	The text of names (Object ID, Frame ID and Frame order).

Variable	Co-ordinates of each object, 4 bytes per point, data added sequentially with no gaps.

~1 kbyte	Alignment and additional plotting parameters (Plotting modes, Translucency, Clipping, etc.)

.c.7.3	Default settings

Selecting .i.Reset 3D; changes the values to .i.Rotation; 30°, Tilt 15°, Distance 0 (i.e. isometric plot) not stereo pair;  Lighting 30° and 0-63 and 64 patterns; Local magnification x1, Layer spacing 0 (i.e. use thickness values from data file), .i.Resolution ;3 (or lower if limited points per object), X-shift 0 and Y-shift 0;  Select Surface plotting mode for all IDs, colours/shades as in table below,  Range of tones for shade by layers 4 to 56;  Order 1 to max. frame;  no additional Linkage, not Aligned, not Translucent, all Goodies selected.

| ID | Shade | Highlight | Shadow | Colour Mac |
| --- | --- | --- | --- | --- |

| 1 | 32 mid grey | 1 red | 3 yellow | orange |
| --- | --- | --- | --- | --- |

| 2 | 16 dark grey | 2 green | 7 white | light green |
| --- | --- | --- | --- | --- |

| 3 | 48 light grey | 4 blue | 6 cyan | light blue |
| --- | --- | --- | --- | --- |

| 4–255 | (ID mod 50)+8 | (ID mod 7) | ((ID+1) mod 7) |  |
| --- | --- | --- | --- | --- |

Highlight and Shadow are translated into black and white with monochrome monitor or 1-bit display.

To display colours, Colour must selected in Mac config. option otherwise Reset 3D will set Highlight  for all IDs to white and Shadow to black.

MacStereology may reverse black and white to make lines show up.

7.4	MacStereology defaults

In addition to options that are saved as part of normal data files, some of the start-up options for MacStereology can be declared within the program’s resources.  New values can be saved by selecting Defaults in File menu.  To change these, it is best to run MacStereology, do not load any data files, then make only the changes required  and finally save them by calling Defaults.

The values include:–

File menu	Calculate refresh, Colour Mac, Grey tones, Drag graphics with mouse, PostScript file options (see Printing on a LaserWriter Section 5.8), File saving options  (Save 3D, Save data – see File options), Window or Full screen.  Tablet active at start up time – see IO Config.

Image menu	Read and analyse options, Binary file options.

Options menu Auto refresh, Step size for tablet, Fill objects, Auto clear, Label objects.

Units menu	Units.

Data menu	Precision  Font name and size.

The keen user can also modify some other aspects of MacStereology such as MENUs, DLOGs and DITLs using a resource editor (.i.ResEdit;), but don’t blame me if it screws the program!  All of the text used for messages is in DITL or STR# resources so can easily be changed., but be careful not to lose leading or trailing spaces or tabs.  Do not change the About Ster dialogue.

It is possible to change the selection of font sizes in MENU 52 and the Applications for Text files in MENU 53 providing their corresponding signatures are put in order in STR# 401.  The order of items in other MENUs cannot be changed.

The resource STR# 400 contains 7 other scale factors:–

The first 2 strings define the x- and y-scale factors for a LaserWriter to give the correct aspect ratio for exact scaling.  By default both are 1.0 (N.B. always use “.” as decimal point).

The third string gives an extra scale factor for the digitising tablet in case its calibration is not exact (default 1.0).  Normally the Magnification would adjust for any errors.

The final 4 strings are integers that convert output from an ADB tablet x = xt x S4/S5, y = yt x S6/S7.

7.5	History

The MacStereology program represents the fifth generation of a program for digitising tablets and 3-D reconstruction.  This work started in the mid 1970s using a DEC PDP-8 computer with analogue inputs from two potentiometers on a tablet.  These were programmed in a now obsolete language Focal, with machine language parts for data collection.  The first version was a simple home-made arrangement with a two-link arm with potentiometers at the ‘shoulder’ and ‘elbow’.  The improved version of this had a single sliding ‘arm’ such that the potentiometers generated the positions as polar co-ordinates (ρ,θ) which were used to calculate areas and perimeters and for serial section reconstruction (J. Physiol. 1977 273 23-24p).

Based on this experience, I obtained a Summagraphics digitising tablet for use with a PDP-11 computer.  The language used was Fortran IV with extensive machine language routines for the critical stages.  In this case the tablet generated the Cartesian co-ordinates (x,y) via a serial line interface.  With the more powerful computer the range of calculated parameters could be increased as well as including many statistical and stereological calculations (Proc. R. Microsc. Soc. 1981 16 120-124).  Since I found most of these extra parameters were never used I have not implemented them in MacStereology.

To make the routines for the digitising tablet more readily available, a simplified version was developed for an Apple 2e computer with TDS digitising tablet.  This was programmed in UCSD Pascal and appropriate machine language parts.  Although now portable, the system lacked the power of the previous version, but it has been successfully used for digitising on-line to an electron microscope, including 3-D reconstruction.

The current version for the Macintosh makes use of its more powerful processor and ample memory.  It is written entirely in LightSpeed Pascal (now called Think Pascal version 4.0), and includes many features from the earlier versions.  The Macintosh is cumbersome to program, since it has to conform with the Macintosh standards for colour, etc. and avoid the peculiarities of the Macintosh Operating System.  Many of the statistical and stereological calculations from the PDP-11 version have not been added, since it is possible to transfer data to other well-known statistical packages on the Macintosh.

The essential feature for all modes of the 3-D display has been to produce a rapid and flexible display.  I can see very little general use of a system that takes minutes or hours to do the reconstruction.  The use of an identifier to label parts of an object as they appear on different layers is an essential feature of the program.  The alternative is to make the distance between sections small enough so that the program can deduce which sections go together.  Not only does this give the operator a lot more work in digitising at least twice as many sections, but the program is still likely to make mistakes and require manual guidance.  MacStereology now includes some of this automatic linking, since the analysis of binary images is much less laborious than using a manual tablet.

MacStereology incorporates a new type of 3-D display not available in previous versions – the ‘Surface’ reconstruction in which the surface of the object is interpolated between layers.  Although this requires more calculation than the simple 3-D plots, I still expect a result in 20 sec or so (Europ. J. Cell Biol. 1989 48 (suppl. 25) 61-64;  J. Microscopy, 1990 157 187-196).  For best graphics you really need to view stereo pairs of reconstructions either as prints 6.25 cm apart or much better projected using polarising spectacles.

With the increasing use of frame grabbers I have now included the analysis binary images.  Although the analysis of such images is quick, obtaining clean and accurate images can be difficult.

Another aspect of MacStereology is that it is written entirely in a high-level language so is relatively easy to modify or expand.  I welcome suggestions from users for changes and additions.

By the way, the name ‘MacStereology’ reflects the original reason for developing the program, although it is now used both for stereology and 3-D reconstructions.  In any case Mac3D is a name that is already a Macintosh application.  In the early days of this program the titles of files were restricted to 4 characters so it was known as plain STER;  a name that you will still see in the About Ster and Ster help options in the  Apple menu.

7.6	Glossary

| .i.Align ;sections | To adjust a stack of sections so the features are the correct position and orientation. This is preferably carried out by putting reference markers in the specimen before cutting sections, but can be done on stored data. |
| --- | --- |

| .i.Area | Area is a measurement made on a 2-D specimen (not to be confused with surface area that can be estimated stereologically for a 3-D structure). |
| --- | --- |

| .i.Binary file | An image in which all pixels are black or white with no grey tones (1-bit image) |
| --- | --- |

| Clipping | Producing a cut-away reconstruction. |
| --- | --- |

| .i.Closed object | A closed object is the profile of an object with forms a closed loop. A section through a 3-D object will give 2-D profiles that are closed objects. It is possible for the boundary to cross over itself, but this may result in a curious value for the measured area. |
| --- | --- |

| .i.Data File | A data file is the information gathered from an application. This is normal saved on a magnetic disk (either the hard disk or a floppy disk) and should be kept with a back up copy in case of failure. A MacStereology data file is the collection of values measured for a set of objects, including areas, perimeters, magnification and experiment title and may include the co-ordinates of the objects. |
| --- | --- |

| Depth shading | The standard shading of a Surface plot is by the orientation of the facets. Adding some Depth shading (Lighting dialogue) may improve the appearance. |
| --- | --- |

| .i.Digitising tablet | This is the input device for the Macintosh and consists of a drawing area (typically 11 or 12 inches square) and a cursor or pen to describe the profiles of the objects. The tablet is sensitive to both the position of the cursor/pen and to any buttons being pressed. To record co-ordinates it is necessary to press one of the buttons. |
| --- | --- |

| .i.Finder | This is the simpler operating environment for a Macintosh and enables you to run a single program that will occupy most of the spare memory. Set the Start up to Finder (Special menu from desktop) if you have 1 Mbyte or less memory on your Macintosh (use About Finder from the  menu). |
| --- | --- |

| .i.Frame | The data from the digitising tablet consists of a series of objects with various parameters, such as area and perimeter. Each frame consists of a collection of related objects, typically being all the measurements on a single photograph, or a single histological section. A frame can also have an identifying number and name. |
| --- | --- |

| Interpolated shading | Facets on a surface plot can be sub-divided to reduce the Mach band effect (i.e. the human eye is very sensitive to small changes of tone). This also permits a smooth change of shading between layers of the reconstruction. |
| --- | --- |

| .i.Layers | The Layers 3-D reconstruction is a simple presentation in which each section of the specimen is shown as a flat layer tilted and stacked to form a 3-D structure. Hidden features are masked and different parts of the structure may be shown in different plotting modes. |
| --- | --- |

| .i.Linkage | Linking objects between frames may be necessary before displaying a complicated Surface 3-D reconstruction. By default a surface is drawn between an object on one layer and an object with the same identifying number on the next layer. It is unnecessary to call Linkage for a Layers or Wireframe reconstruction. |
| --- | --- |

| .i.Multifinder | The Multifinder is the more powerful operating environment for a Macintosh that enables you to run more than one application at the same time, but can only sensibly be used if you have at least 2 Mbyte memory. |
| --- | --- |

| .i.Object | An object is the profile of a structure that can be described with the cursor on the digitising tablet. It may be a closed object having both an area and perimeter or an open object with only a boundary length. The data file saves these values, and also has an identifying number (object ID) and a name. The ID is a flag that can be used to classify your data or to indicate which parts of a 3-D structure belong together. |
| --- | --- |

| .i.Open object | An open object is one in which the boundary has a start and end, but does not form a closed loop. This may be a length of string or a strand of DNA; it can cross over itself or other objects. |
| --- | --- |

| .i.Perimeter | The perimeter of an object is the length of its boundary (whether an open or closed object). Measurements of perimeter are sensitive to the Step size since the fine detail (or hand shake) increase the measured perimeter if a small Step size is used. |
| --- | --- |

| .i.Pict files | File of graphics commands that can be read by many Macintosh applications. These can be lines and shapes (MacDraw type), binary images (MacPaint), grey tone or colour images (PhotoShop). |
| --- | --- |

| Pixel | A single point on the screen. There is some confusion whether the pixel is infinitesimal small or is a small blob, hence the choice of Chunky pixels in the analysis of binary images. |
| --- | --- |

| .i.PostScript | A language used by graphics devices such as the LaserWriter. Postscript files are text files that can (with great care) be edited using a simple word processor. Postscript files from MacStereology use only a few commands and make use of the high resolution of the LaserWriter and its ability to print half tone shade. |
| --- | --- |

| .i.Stereology | The technique of estimating average 3-D properties of a structure from measurements taken on 2-D sections. In order to obtain statistically valid results it is essential to use the precisely defined criteria before applying stereological formulæ. The efficiency of measurements, which can be very laborious, can be improved by choosing appropriate systematic sampling procedures. |
| --- | --- |

| .i.Solid | A solid plotting mode for a 3-D reconstruction is one in which one layer completely masks the layer behind. Features may be totally obscured in a solid plot, but the boundary can be seen using the line+dots and line+dashes modes. |
| --- | --- |

| .i.Step size | The Step size refers to the minimum distance between data points when using a digitising tablet. The normal value would be 0.5 to 2 mm. The reason for a step size is to make the data points approximately evenly spaced and to reduce the number of data points collected. This spacing can be seen when a 3-D reconstruction is displayed using one of the dotted or dashed modes. |
| --- | --- |

| .i.Surface | A Surface 3-D reconstruction puts a computed surface between the parts of a structure as it occurs on adjacent serial sections. By default this surface is drawn between objects of the same identifying number, but different objects can be Linked, which will enable branches and joins to be specified. |
| --- | --- |

| .i.System error | If the program crashes irretrievably the Macintosh system will usually put up a ‘bomb’ or MultiFinder error message with an error number. Error messages in System 7 are a little more friendly. If you are collecting data and have a file open it will have been already saved up to the last End frame. |
| --- | --- |

| .i.Text file | These files can be generated by MacStereology for transferring data to other applications. These files can be read by a word processor (e.g. MacWrite) but are more likely to be used by statistical and graphics packages. The text files can be either the co-ordinates of a single object, a summary of the measured values for all or part of a data file or graphics commands in the Postscript language. |
| --- | --- |

| Translucency | It is possible to make a reconstruction in which hidden features can be seen through the masking object. The degree of translucency can be varied from Lighting dialogue and this can be Selected for any ID. |
| --- | --- |

| .i.Wireframe | A Wireframe 3-D reconstruction is the simplest display with only lines joining the digitised points and masking of hidden features. However, it can be used with a digital plotter since the masking is precalculated, unlike the Layers and Surface plots that rely on the screen mapping to erase hidden features. Wireframe displays features in Surface plotting mode as Solid mode. |
| --- | --- |

Macintosh is a registered trademark of Macintosh Laboratory Inc., Apple, LaserWriter, Appletalk and  MacWrite are trademarks of Apple Computers Inc., Summagraphics and MacTablet are trademarks of Summagraphics Corp.,  LightSpeed Pascal and Think Pascal are trademarks of Think Technologies Inc. and Symantec Corp., SuperLaserSpool is a trademark of SuperMac Software.  This manual is written in Times font using Microsoft Word 4.0.

The original Bit Pad Plus is not satisfactory and may result in slow input.  Follow the recommended procedure of plugging in the digitising tablet before power up of the Macintosh, but do not install the Bit Pad Plus Driver, since the tablet cannot then be reconfigured for use by MacStereology.  The supplied Summagraphics software can apparently successfully configure the digitising tablet, even if the power switch on the tablet is switched off, but  obviously the tablet will not work!  The current configuration of the Bit Pad Plus is saved when it is in use by MacStereology and restored on exit.

* Images read from Pict files use the system colour table, whereas 3-D reconstructions have their own indexed colour to enable them to show just the ranges of colour used by the reconstruction.  A 3-D reconstruction is saved with this indexed colour table and can be read by many other applications in its original colours, but some applications convert all colour images back to the system palette, giving a rather crude representation of the colours.  If Colour/grey tones is turned off the reconstruction uses only the 8 basic colours and produces shades using dither patterns.

* Input from a digitising tablet via the ADB port sometimes gets confused by either MacStereology losing the input (cured by calling IO config again) or leaving one of its little crosses on screen (data not affected).  The correct tablet size and configuration cannot always be read (this may be cured by calling the tablet configuration dialogue in the Control panel or turning the tablet power off and on, but it may be necessary to Restart the system as the tablet should be plugged in before restarting/switching on).

* This is not implemented with the old Summagraphics ADB tablet, but Edit data (Data menu) can be used to convert an Open profile to points.

* If the Recalc refresh is selected in Mac config (File menu) dragging moves the outline of the window.  Since you are dragging the window, to move the display left, drag the window to the right.

* If Recalculating refresh the frame round the screen will be moved, not the image.

* A system error in system 6 and 7 means that the dates and sizes of files created or modified are not shown correctly in folders until the folder/disk has been closed then reopened.  Be careful when backing up data not to miss a new or modified file which still has an old date.

* A system error in 6.0.7 and 7 means that pop-up menus in dialogues may not have the correct frame if they touch the top or bottom of the screen.  They work correctly, but you may see blank spaces.

* Not implemented with the old Summagraphics ADB tablet, but Edit data (Data menu) can be used to convert an open profile to points.